NetSDK Programming Manual

VERSION 3.0.5.4 (Build 20120605)

2012-06-05

All rights reserved

Foreword

Thank you for using our devices. We are going to provide best service for you. This manual may contain spelling grammar and punctuation errors.We will update this manual regularly.

Modification History

| Date | Content |
| --- | --- |
| 2008.10.22 | Create |
| 2009.02.12 | Add H264_DVR_SearchDevice for search device at LAN |
| 2009.02.18 | Add API with talk: H264_DVR_StartVoiceCom_MR，H264_DVR_VoiceComSendData，H264_DVR_StopVoiceCom，H264_DVR_SetTalkMode |
| 2009.09.26 | Add API below: H264_DVR_StartDVRRecord，H264_DVR_StopDVRRecord，H264_DVR_SetSystemDateTime， H264_DVR_GetDVRWorkState， H264_DVR_ClickKey |
| 2011.07.14 | Add API with DVR color setting H264_DVR_GetDevConfig(E_SDK_VIDEOCOLOR，…) H264_DVR_SetDevConfig(E_SDK_VIDEOCOLOR，…) SDK_CONFIG_VIDEOCOLOR Add get deviceInfo from ddns Server H264_DVR_GetDDNSInfo（） |
| 2011.08.01 | H264_DVR_OpenTransComChannel H264_DVR_CloseTransComChannel |
| 2011.09.06 | Add API Below: H264_DVR_OpenTransComChannel，H264_DVR_CloseTransComChannel，H264_DVR_GetDeviceState, H264_DVR_SetConnectTime,H264_DVR_SetConfigOverNet,H264_DVR_PauseRealPlay, H264_DVR_SetRealDataCallBack_V2,H264_DVR_FindFileByTime,H264_DVR_PlayBackByName_V2,H264_DVR_PlayBackByTime,H264_DVR_PlayBackByTimeEx,H264_DVR_GetFileByTime,H264_DVR_CatchPic,H264_DVR_CatchPicInBuffer, H264_DVR_SerialWrite, H264_DVR_SerialRead, H264_DVR_MakeKeyFrame |
| 2011.09.27 | Add Data Struct: SDK_AllCameraParam, SDK_CameraAbility |
| 2011.10.10 | Add api: H264_DVR_CatchPicUI |
| 2011.11.21 | Add api:(Combin net and decod ) H264_DVR_OpenSound H264_DVR_CloseSound H264_DVR_LocalCatchPic H264_DVR_StartLocalVoiceCom H264_DVR_StartLocalRecord H264_DVR_StopLocalRecord H264_DVR_StartLocalPlay H264_DVR_StopLocalPlay H264_DVR_GetPlayPos H264_DVR_SetPlayPos H264_DVR_LocalPlayCtrl H264_DVR_SetFileEndCallBack H264_DVR_SetInfoFrameCallBack H264_DVR_LocalGetColor H264_DVR_LocalSetColor Add struct: SDK_LoalPlayAction Chang struct: (add hWnd ins struct) H264_DVR_FILE_DATA H264_DVR_FINDINFO H264_DVR_CLIENTINFO |
| 2012.05.18 | 增加以下接口 H264_DVR_SetLocalBindAddress H264_DVR_StartUploadData H264_DVR_StopUploadData H264_DVR_StartActiveRigister H264_DVR_StopActiveRigister H264_DVR_SetSubDisconnectCallBack ConnectStyle Enum ：SocketStyle ActiveConnect Struct ：H264_DVR_ACTIVEREG_INFO |
| 2012.8.2 | 增加以下接口 H264_DVR_SetKeepLifeTime |

Catalogue

1. General Introduction	8

1.1 Introduction	8

1.2 Applicability	8

Desig of principia	9

1.3 Programming Description	9

1.4 API Calling Reference	10

2 Data Structure Description	12

2.1 Client Data Structure Description	12

2.1.1 Common Defination	15

2.1.2 Structure of device information	23

2.1.3 Date Information	24

2.1.4 Record File Information	24

2.1.5 Structure of Configuration	29

2.1.6 Network keyboard define	51

2.1.7 Network Alarm Information	53

2.1.8 Storage Device Control Information	55

2.1.9 RTSP	55

2.1.10 MECG platform	56

2.1.11 XINWANG platform	56

2.1.12 SHISOU platform	57

2.1.13 VVEYE platform	57

2.1.14 Media Packet Information	57

2.1.15 LocalPlay control	61

2.1.16 Active Register Server	61

2.1.17 SubConnectStyle	61

2.1.18 ConnectStyle	62

3 API Definition	63

3.1 SDK Initialization	63

3.2 Get alarm status	64

3.3 Device Registration	66

3.4 Real-time Monitor	67

3.5 Playback And Download	70

3.6 Playback Control	76

3.7 PTZ Control	77

3.8 System Configuration	78

3.9 Log Management	79

3.10 Remote Control	80

3.11 Upgrade	80

3.11 Audio Talk	82

3.12 Record Mode	84

3.13 Set System Time	84

3.14 Get Device status	85

3.15 Net Keyboard	85

3.16 NetWork Alarm	85

3.17 AlarmCenter	86

3.18 DiskManager	86

3.19 Capture	87

3.20 Pass through channel 232,485	88

3.21 Get device information from DDNS	89

3.22 Enforcement I frame	89

3.23 Setting connect timeout and retry  times	90

3.24 Open transparent channel	90

3.25 Close transparent channel	91

3.26 Get Device Status	91

3.27 Get user operate interface	92

3.28 Client-side Recording	92

3.29 Open talking（2）	93

3.30 Client-side Audio	93

3.31 Client-side cath picture	94

3.32 Playing Position	94

3.33 Set information frame callback	95

3.34 The Color of Client-side	95

3.35 Play Client-side file	97

3.36 Binding local IP	98

3.37 Upload data set callbacks	99

3.38 Support advice to Active register	100

3.39 Set the sub connection broken callback	100

3.40 Set time interval of heartbeat and time interval of disconnect	101

# General Introduction

## Introduction

This SDK is for  DVR,network video server and etc. The text describes functions and interfaces as well as relations among them.It also provide detailed demonstration.

This package includes two parts: operation and equipment management：

Operation

state listening, real-time monitor, real-time preview, character overlay, audio control, record playback and download, data storage, PTZ control, alarm deployment, voice dialogue, log management, user management, action on alarm, transparent com port and etc.

Device management

remote upgrade, remote restart/shutdown, equipment parameters setup such as general setting, alarm, record, serial port, image, log, user management, device timing, motion detection, network and etc.

The development package includes the following files：

| Network library | NetSDK | Head file |
| --- | --- | --- |
|  | NetSDK.lib | Lib file |
|  | NetSDK.dll | Interface library |
| Assitante library | H264Play.dll | Decode assistant library |
|  | DllDeinterlace.dll | Decode assistant library |
|  | hi_h264dec_w.dll | Decode assistant library |

## Applicability

- Support real-time monitor playback,、Alarm、Remote config、log management and etc。

- Support TCP network tramsmisson, Max 10 TCP connections

- You can use this SDK to connect to device to develop client-end program, or you can use SDK callback interface to develop server program such as stream media transfer, playback, alarm and etc.

- The client supports multi- image resolutions including QCIF、CIF、2CIF、HalfD1、D1，VGA（640×480）and etc.

- When SDK is playback/download, one logged  ID can not operate playback and downalod for the same channel at the same time.

- SDK performance has relationship woth device running status and client-end PC capability. Basically, it can support 1024 users to register at the same time, 1024-ch network preview and playback at the same time. Support 1024-ch alarm upload at the same time .And 100-ch video display

# Desig of principia

## Programming Description

- Initialization and clean up

- 1、First calling H264_DVR_Init() to initialize SDK，when Application exit, calling H264_DVR_Cleanup()to release all occupied resource.

- 2、 Most API shall call after H264_DVR_Init(), before H264_DVR_Cleanup() ，But H264_DVR_GetLastError can be called anywhere.

- Login in and login out

- Befor access device, you shall call H264_DVR_Login() login first, also you can call H264_DVR_LoginEx() to assign your client type logined. If login succeeded, will return a global handle, the handle is a session channel, you can use it to operate device. Call H264_DVR_Logout() to close this session.

- Callback Fuction

- Callback function have a parameter: dwUser, it is elf-define parameter, you can define your owner data.

## API Calling Reference

The below diagram describes a brief API calling reference for basic client application, and users can add in other function modules according to actual application requirements.

- Initialization

SDK Initialization H264_DVR_Init ()

- Set callback for alarm message

Set callback H264_DVR_SetDVRMessCallBack ()

- Login in

Login in H264_DVR_Login () H264_DVR_LoginEx () Subscribe for alarm message H264_DVR_SetupAlarmChan ()

- Operation of device ,get and set parameter configuration

Parameter configuration H264_DVR_GetDevConfig () H264_DVR_SetDevConfig () Log searching H264_DVR_FindDVRLog () PTZ control H264_DVR_PTZControl () H264_DVR_PTZControlEx () TransCom H264_DVR_OpenTransComChannel（） H264_DVR_CloseTransComChannel（）

- Real-time preview

Open and close monitor channel H264_DVR_RealPlay () H264_DVR_StopRealPlay () Callback for saving monitor data H264_DVR_SetRealDataCallBack ()

- Playback/Download

Record Searching H264_DVR_FindFile () H264_DVR_FindFileByTime() Playback control H264_DVR_PlayBackByName() H264_DVR_PlayBackByName_V2() H264_DVR_PlayBackByTime() H264_DVR_PlayBackByTimeEx() H264_DVR_PlayBackControl() H264_DVR_StopPlayBack() Download H264_DVR_GetFileByName () H264_DVR_GetFileByTime() H264_DVR_GetDownloadPos() H264_DVR_StopGetFile ()

- Remote control

Upgrade H264_DVR_Upgrade() H264_DVR_GetUpgradeState() H264_DVR_CloseUpgradeHandle() Reboot /Clear log H264_DVR_ControlDVR（）

- Login out

Cancel subscribe for alarm message H264_DVR_CloseAlarmChan () Disconnect H264_DVR_Logout ()

- Release SDK resource

SDK exit H264_DVR_Cleanup ()

# Data Structure Description

## Client Data Structure Description

//PTZ control type

typedef enum PTZ_ControlType

{

TILT_UP = 0,			//UP

TILT_DOWN,			//DOWN

PAN_LEFT,				//LEFT

PAN_RIGTH,			//RIGTH

PAN_LEFTTOP,			//LEFT TOP

PAN_LEFTDOWN,			//LEFT DOWN

PAN_RIGTHTOP,			//RIGTH TOP

PAN_RIGTHDOWN,		//RIGTH DOWN

ZOOM_IN,				//ZOOM IN

ZOOM_OUT,				//ZOOM OUT

FOCUS_FAR,			//FOCUS FAR

FOCUS_NEAR,			//FOCUS NEAR

IRIS_OPEN,			//IRIS OPEN

IRIS_CLOSE,			//IRIS CLOSE

EXTPTZ_OPERATION_ALARM,			//ALARM

EXTPTZ_LAMP_ON,					//LIGTH OPEN

EXTPTZ_LAMP_OFF,					//LIGTH CLOSE

EXTPTZ_POINT_SET_CONTROL,		//SET PRESET POINT

EXTPTZ_POINT_DEL_CONTROL,		//CLEAR PRESET POINT

EXTPTZ_POINT_MOVE_CONTROL,		//GOTO PRESET POINT

EXTPTZ_STARTPANCRUISE,			//START PAN CRUISE

EXTPTZ_STOPPANCRUISE,			//STOP PAN CRUISE	EXTPTZ_SETLEFTBORDER,			//SET LEFT BORDER

EXTPTZ_SETRIGHTBORDER,			//SET RIGHT BORDER

EXTPTZ_STARTLINESCAN,			//START AUTO SCAN

EXTPTZ_CLOSELINESCAN,			//STOP AUTO SCAN

EXTPTZ_ADDTOLOOP,				//ADD PRESET POINT TO CRUISE LINE

EXTPTZ_DELFROMLOOP,				//DEL PRESET POINT FROM CRUISE LINE

EXTPTZ_POINT_LOOP_CONTROL,		//START CRUISE

EXTPTZ_POINT_STOP_LOOP_CONTROL,//STOP CRUISE

EXTPTZ_CLOSELOOP,				//CLEAR CRUISE LINE

EXTPTZ_FASTGOTO,					//FAST GOTO

EXTPTZ_AUXIOPEN,				//AUX OPEN

EXTPTZ_OPERATION_MENU,		//OPERATION MENU

EXTPTZ_REVERSECOMM,			//REVER CAMERAL

EXTPTZ_OPERATION_RESET,		///< PTZ RESET

EXTPTZ_TOTAL,

};

Error code，return by GetLastError

typedef enum SDK_RET_CODE

{

H264_DVR_NOERROR					= 0,		//no error

H264_DVR_SUCCESS					= 1,		//success

H264_DVR_SDK_NOTVALID			= -10000,	//invalid request

H264_DVR_NO_INIT					= -10001,	//SDK not inited

H264_DVR_ILLEGAL_PARAM			= -10002,	// illegal user parameter

H264_DVR_INVALID_HANDLE			= -10003,	//handle is null

H264_DVR_SDK_UNINIT_ERROR		= -10004,	//SDK clear error

H264_DVR_SDK_TIMEOUT				= -10005,	//timeout

H264_DVR_SDK_MEMORY_ERROR		= -10006,	//memory error	H264_DVR_SDK_NET_ERROR			= -10007,	//network error

H264_DVR_SDK_OPEN_FILE_ERROR	= -10008,	//open file fail

H264_DVR_SDK_UNKNOWNERROR		= -10009,	//unknown error

H264_DVR_DEV_VER_NOMATCH			= -11000,	//version mismatch

H264_DVR_ERROR_GET_DATA			= -11001,	//get data fail（including configure, user information and etc）

H264_DVR_OPEN_CHANNEL_ERROR		= -11200,	//open channel fail

H264_DVR_CLOSE_CHANNEL_ERROR	= -11201,	//close channel fail

H264_DVR_SUB_CONNECT_ERROR		= -11202,	//open media connet fail	H264_DVR_SUB_CONNECT_SEND_ERROR	= -11203,	//media connet send data fail

/// error code of user management

H264_DVR_NOPOWER					= -11300,	//no power

H264_DVR_PASSWORD_NOT_VALID		= -11301,	// password not valid

H264_DVR_LOGIN_USER_NOEXIST		= -11302,	// user not exist

H264_DVR_USER_LOCKED				= -11303,	// user is locked

H264_DVR_USER_IN_BLACKLIST		= -11304,	// user is in backlist

H264_DVR_USER_HAS_USED			= -11305,	// user have logined	H264_DVR_USER_NOT_LOGIN			= -11305,	// no login	H264_DVR_CONNECT_DEVICE_ERROR   = -11306,	// maybe device no exist

/// error code of configure management

H264_DVR_OPT_RESTART				= -11400,	// need to restart application	H264_DVR_OPT_REBOOT				= -11401,	// need to reboot system	H264_DVR_OPT_FILE_ERROR			= -11402,	// write file fail

H264_DVR_OPT_CAPS_ERROR			= -11403,	// not support

H264_DVR_OPT_VALIDATE_ERROR		= -11404,	// validate fail	H264_DVR_OPT_CONFIG_NOT_EXIST	= -11405,	// config not exist

H264_DVR_CTRL_PAUSE_ERROR		= -11500,	// pause fail

H264_DVR_SDK_NOTFOUND			= -11501,			//not found

H264_DVR_CFG_NOT_ENABLE         = -11502,           //cfg not enable

};

- Alarm Event Type

enum SDK_EventCodeTypes

{

SDK_EVENT_CODE_INIT = 0,

SDK_EVENT_CODE_LOCAL_ALARM = 1,	// local alarm	SDK_EVENT_CODE_NET_ALARM,			// network alarm	SDK_EVENT_CODE_MANUAL_ALARM,		// manual alarm

SDK_EVENT_CODE_VIDEO_MOTION,		// motion detect

SDK_EVENT_CODE_VIDEO_LOSS,			// loss detect	SDK_EVENT_CODE_VIDEO_BLIND,		// blind detect

SDK_EVENT_CODE_VIDEO_TITLE,

SDK_EVENT_CODE_VIDEO_SPLIT,

SDK_EVENT_CODE_VIDEO_TOUR,

SDK_EVENT_CODE_STORAGE_NOT_EXIST,

SDK_EVENT_CODE_STORAGE_FAILURE,

SDK_EVENT_CODE_LOW_SPACE,

SDK_EVENT_CODE_NET_ABORT,

SDK_EVENT_CODE_COMM,

SDK_EVENT_CODE_STORAGE_READ_ERROR,

SDK_EVENT_CODE_STORAGE_WRITE_ERROR,

SDK_EVENT_CODE_NET_IPCONFLICT,

SDK_EVENT_CODE_ALARM_EMERGENCY,

SDK_EVENT_CODE_DEC_CONNECT,

SDK_EVENT_CODE_NR,

};

//alarm information

typedef struct SDK_ALARM_INFO

{

int nChannel;

int iEvent;  ///< refer to SDK_EventCodeTypes

int iStatus; ///< 0: start 1: stop

SDK_SYSTEM_TIME SysTime;

}SDK_AlarmInfo;

### Common Defination

#define PAN_AUTO		29

#define EXTPTZ_FASTGOTO	30

#define NET_MAX_CHANNUM 			32			//max numbers of channels

#define NET_DECORDR_CH              16          //max numbers of decorder channels

#define NET_MAX_USER_NUM			60			//max numbers of users

#define NET_MAX_RIGTH_NUM			128			//max items of power

#define NET_MAX_GROUP_NUM			50			//max numbers of groups

#define NET_MAX_USER_LENGTH			32			//max length of username

#define NET_MAX_COMBINE_NUM			2			//max numbers of combine-encode channel

#define NET_MAX_DECORDR_CH          16          //max channel of decoder

#define NET_MAX_DDNS_TYPE			5			//max types of DDNS support

#define NET_MAX_ARSP_TYPE			5

#define NET_MAX_ALARMSERVER_TYPE    5

#define NET_MAX_SYSFUNC				20			//max system function

#define NET_MAX_PTZ_PROTOCOL_LENGTH 32			//max protocol name length

#define NET_N_WEEKS					7			//weeks

#define	NET_N_TSECT					6			//time setct numbers

#define NET_MD_REGION_ROW			32			//row numbers of motion detect region

#define NET_COVERNUM				8			//max numbers of video cover

#define NET_MAX_FILTERIP_NUM		64			//max numbers of IP fliter

#define NET_NAME_PASSWORD_LEN		64			//max length of password

#define NET_MAX_PATH_LENGTH	        260			//max length of path

#define NET_N_MIN_TSECT				2

#define NET_MAX_RETURNED_LOGLIST	128			//max numbers of log

#define NET_MAX_MAC_LEN             32      //max length of mac addr

#define NET_IW_ENCODING_TOKEN_MAX   128

#define NET_MAX_AP_NUMBER           10

#define NET_MAX_INFO_LEN		    128

#define NET_MAX_USERNAME_LENGTH		128

#define NET_MAX_SERIALNO_LENGTH		128          //Max serial no. length

//DDNS

#define DDNS_MAX_DDNS_NAMELEN 64  //DDNS hostname len

#define DDNS_MAX_DDNS_PWDLEN 32   //password len

#define DDNS_MAX_DDNS_IPSIZE 64   //IP address len

#define DDNS_MAX_DDNS_IDSIZE 32   //Device's ID len

#define DDNS_MAX_SERIALINFO_SIZE 16 //Serial ID len or Username len

//Camera Parameter

#define CAMERAPARA_MAXNUM  16    //Max Length

typedef struct _DDNS_INFO

{

char ID[64];    //dev id

char IP[DDNS_MAX_DDNS_IPSIZE];   //local IP

int WebPort;                     //HTTP port

int MediaPort;                   //Media port

int MobilePort;                  //mobile port

int UPNPWebPort;                 //HTTP Port after UPNP

int UPNPMediaPort;               //media Port after UPNP

int UPNPMobilePort;              //moile Port after UPNP

char Username[DDNS_MAX_DDNS_NAMELEN];  //username

char Password[DDNS_MAX_DDNS_PWDLEN];   //password

}DDNS_INFO,*pDDNS_INFO;

//DDNS Search

typedef struct _SearchMode

{

int  nType;        //Type，见SearchModeType

char szSerIP[DDNS_MAX_DDNS_IPSIZE];//Server address

int nSerPort;           //Port

char szSerialInfo[DDNS_MAX_SERIALINFO_SIZE];  //if it is Serial ID ,it will be Serial ID;if it is Username,it will be Username

}SearchMode;

//DDNS Search TYPE

enum SearchModeType

{

DDNS_SERIAL = 0,//By Serial ID

DDNS_USERNAME,  //By Username

};

typedef enum _H264_DVR_RealPlayType

{

NET_RType_Realplay = 0,	//main stream 1 monitor

NET_RType_Main_Realplay2, //main stream 2 monitor

NET_RType_Main_Realplay3, //main stream 3 monitor

NET_RType_Sub_Realplay_1,	//extra stream 1

NET_RType_Sub_Realplay_2,	//extra stream 2

NET_RType_Sub_Realplay_3,	//extra stream 3

NET_RType_Multiplay_1,	//

NET_RType_Multiplay_4,	//

NET_RType_Multiplay_6,	//

NET_RType_Multiplay_8,	//

NET_RType_Multiplay_9,	//

NET_RType_Multiplay_12,	//

NET_RType_Multiplay_16,	//

} NET_RealPlayType;

/// combine-encode type

enum NetCombinType

{

NET_COMBIN_NONE,

NET_COMBIN_1,

NET_COMBIN_2,

NET_COMBIN_3,

NET_COMBIN_4,

NET_COMBIN_5,

NET_COMBIN_6,

NET_COMBIN_7,

NET_COMBIN_8,

NET_COMBIN_9,

NET_COMBIN_10,

NET_COMBIN_11,

NET_COMBIN_12,

NET_COMBIN_13,

NET_COMBIN_14,

NET_COMBIN_15,

NET_COMBIN_16,

NET_COMBIN_1_4,

NET_COMBIN_5_8,

NET_COMBIN_9_12,

NET_COMBIN_13_16,

NET_COMBIN_1_8,

NET_COMBIN_9_16,

NET_COMBIN_1_9,

NET_COMBIN_8_16,

NET_COMBIN_1_16

};

// encode resolution

enum SDK_CAPTURE_SIZE_t {

SDK_CAPTURE_SIZE_D1,		///< 720*576(PAL)	720*480(NTSC)

SDK_CAPTURE_SIZE_HD1,		///< 352*576(PAL)	352*480(NTSC)

SDK_CAPTURE_SIZE_BCIF,		///< 720*288(PAL)	720*240(NTSC)

SDK_CAPTURE_SIZE_CIF,		///< 352*288(PAL)	352*240(NTSC)

SDK_CAPTURE_SIZE_QCIF,		///< 176*144(PAL)	176*120(NTSC)

SDK_CAPTURE_SIZE_VGA,		///< 640*480(PAL)	640*480(NTSC)

SDK_CAPTURE_SIZE_QVGA,		///< 320*240(PAL)	320*240(NTSC)

SDK_CAPTURE_SIZE_SVCD,		///< 480*480(PAL)	480*480(NTSC)

SDK_CAPTURE_SIZE_QQVGA,		///< 160*128(PAL)	160*128(NTSC)

SDK_CAPTURE_SIZE_ND1 = 9,     ///< 240*192

SDK_CAPTURE_SIZE_650TVL,      ///< 926*576

SDK_CAPTURE_SIZE_720P,        ///< 1280*720

SDK_CAPTURE_SIZE_1_3M,        ///< 1280*960

SDK_CAPTURE_SIZE_UXGA ,       ///< 1600*1200

SDK_CAPTURE_SIZE_1080P,       ///< 1920*1080

SDK_CAPTURE_SIZE_WUXGA,       ///< 1920*1200

SDK_CAPTURE_SIZE_2_5M,        ///< 1872*1408

SDK_CAPTURE_SIZE_3M,          ///< 2048*1536

SDK_CAPTURE_SIZE_5M,          ///< 3744*1408

SDK_CAPTURE_SIZE_NR,

};

/// stream control type

enum SDK_capture_brc_t {

SDK_CAPTURE_BRC_CBR,			///< Constant bitrate

SDK_CAPTURE_BRC_VBR,			///< variable bitrate

SDK_CAPTURE_BRC_MBR,			///< mix bitrate

SDK_CAPTURE_BRC_NR				///<

};

/// encode format

enum SDK_CAPTURE_COMP_t {

SDK_CAPTURE_COMP_DIVX_MPEG4,	///< DIVX MPEG4。

SDK_CAPTURE_COMP_MS_MPEG4,		///< MS MPEG4。

SDK_CAPTURE_COMP_MPEG2,			///< MPEG2。

SDK_CAPTURE_COMP_MPEG1,			///< MPEG1。

SDK_CAPTURE_COMP_H263,			///< H.263

SDK_CAPTURE_COMP_MJPG,			///< MJPG

SDK_CAPTURE_COMP_FCC_MPEG4,		///< FCC MPEG4

SDK_CAPTURE_COMP_H264,			///< H.264

SDK_CAPTURE_COMP_NR				///<

};

/// stream type

enum SDK_CAPTURE_CHANNEL_t {

SDK_CHL_MAIN_T = 0,		///< main stream

SDK_CHL_2END_T = 1,		///< extram stream 1

SDK_CHL_3IRD_T = 2,		///< extram stream 2

SDK_CHL_4RTH_T = 3,		///< extram stream 3

SDK_CHL_JPEG_T = 4,		///< snapshot

SDK_CHL_FUNCTION_NUM

};

/// audio format

enum SDK_AudioEncodeTypes

{

SDK_AUDIO_ENCODE_NONE = 0,

SDK_AUDIO_ENCODE_G729_8KBIT,

SDK_AUDIO_ENCODE_G726_16KBIT,

SDK_AUDIO_ENCODE_G726_24KBIT,

SDK_AUDIO_ENCODE_G726_32KBIT,

SDK_AUDIO_ENCODE_G726_40KBIT,

SDK_AUDIO_ENCODE_PCM_8TO16BIT,

SDK_AUDIO_ENCODE_PCM_ALAW,

SDK_AUDIO_ENCODE_PCM_ULAW,

SDK_AUDIO_ENCODE_ADPCM8K16BIT,

SDK_AUDIO_ENCODE_ADPCM16K16BIT,

SDK_AUDIO_ENCODE_G711_ALAW,

SDK_AUDIO_ENCODE_MPEG2_LAYER1,

SDK_AUDIO_ENCODE_AMR8K16BIT,

SDK_AUDIO_ENCODE_G711_ULAW,

SDK_AUDIO_ENCODE_IMA_ADPCM_8K16BIT,

SDK_AUDIO_ENCODE_TYPES_NR,

};

/// event code

enum SDK_EventCodeTypes

{

SDK_EVENT_CODE_INIT = 0,

SDK_EVENT_CODE_LOCAL_ALARM = 1,	//local alarm

SDK_EVENT_CODE_NET_ALARM,		//network alarm

SDK_EVENT_CODE_MANUAL_ALARM,	//manual alarm

SDK_EVENT_CODE_VIDEO_MOTION,	//motion detect

SDK_EVENT_CODE_VIDEO_LOSS,		//loss detect

SDK_EVENT_CODE_VIDEO_BLIND,		//blind detect

SDK_EVENT_CODE_VIDEO_TITLE,

SDK_EVENT_CODE_VIDEO_SPLIT,

SDK_EVENT_CODE_VIDEO_TOUR,

SDK_EVENT_CODE_STORAGE_NOT_EXIST,

SDK_EVENT_CODE_STORAGE_FAILURE,

SDK_EVENT_CODE_LOW_SPACE,

SDK_EVENT_CODE_NET_ABORT,

SDK_EVENT_CODE_COMM,

SDK_EVENT_CODE_STORAGE_READ_ERROR,

SDK_EVENT_CODE_STORAGE_WRITE_ERROR,

SDK_EVENT_CODE_NET_IPCONFLICT,

SDK_EVENT_CODE_ALARM_EMERGENCY,

SDK_EVENT_CODE_DEC_CONNECT,

SDK_EVENT_CODE_NR,

};

// Record type

enum SDK_ENCODE_TYPE_BY_RECORD

{

SDK_ENCODE_TYPE_TIM = 0, //time

SDK_ENCODE_TYPE_MTD = 1, //motion detect

SDK_ENCODE_TYPE_ALM = 2, //alarm in

SDK_ENCODE_TYPE_NUM = 3,

SDK_ENCODE_TYPE_SNAP_TIMER = 0,

SDK_ENCODE_TYPE_SNAP_TRIGGER = 1,

};

#define SDK_EXTRATYPES 3 //extra types

//stream transfers policy

enum SDK_TransferPolicy

{

SDK_TRANSFER_POLICY_AUTO,		///< auto

SDK_TRANSFER_POLICY_QUALITY,	///< quality first

SDK_TRANSFER_POLICY_FLUENCY,	///< fluency first

SDK_TRANSFER_POLICY_NR,

};

/// encode ability

enum SDK_EncodeFunctionTypes

{

SDK_ENCODE_FUNCTION_TYPE_DOUBLE_STREAM,		///< multi-stream

SDK_ENCODE_FUNCTION_TYPE_COMBINE_STREAM,	///< combine-encode

SDK_ENCODE_FUNCTION_TYPE_SNAP_STREAM,		///< snapshot

SDK_ENCODE_FUNCTION_TYPE_NR,

};

/// Alarm Function

enum SDK_AlarmFucntionTypes

{

SDK_ALARM_FUNCTION_TYPE_MOTION_DETECT,	///< motion detect

SDK_ALARM_FUNCTION_TYPE_BLIND_DETECT,	///< blind detect

SDK_ALARM_FUNCTION_TYPE_LOSS_DETECT,	///< loss detect

SDK_ALARM_FUNCTION_TYPE_LOCAL_ALARM,	///< local alarm

SDK_ALARM_FUNCTION_TYPE_NET_ALARM,		///< net alarm

SDK_ALARM_FUNCTION_TYPE_IP_CONFLICT,	///< ip conflict

SDK_ALARM_FUNCTION_TYPE_NET_ABORT,		///< net abort

SDK_ALARM_FUNCTION_TYPE_STORAGE_NOTEXIST,	///< storage not exist

SDK_ALARM_FUNCTION_TYPE_STORAGE_LOWSPACE,	///< low space

SDK_ALARM_FUNCTION_TYPE_STORAGE_FAILURE,	///< access storage fail

SDK_ALARM_FUNCTION_TYPE_NR

};

/// net server

enum SDK_NetServerTypes

{

SDK_NET_SERVER_TYPES_IPFILTER,		///< IP fliter

SDK_NET_SERVER_TYPES_DHCP,			///< DHCP

SDK_NET_SERVER_TYPES_DDNS,			///< DDNS

SDK_NET_SERVER_TYPES_EMAIL,			///< Email

SDK_NET_SERVER_TYPES_MULTICAST,		///< multi-cast

SDK_NET_SERVER_TYPES_NTP,			///< NTP

SDK_NET_SERVER_TYPES_PPPOE,         ///< PPPOE

SDK_NET_SERVER_TYPES_DNS,           ///< DNS

SDK_NET_SERVER_TYPES_ARSP,			///< ARSP

SDK_NET_SERVER_TYPES_3G,            ///< 3G

SDK_NET_SERVER_TYPES_MOBILE,        ///< Mobile monitor

SDK_NET_SERVER_TYPES_UPNP,			    ///< UPNP

SDK_NET_SERVER_TYPES_FTP,			    ///< FTP

SDK_NET_SERVER_TYPES_WIFI,          ///<WIFI

SDK_NET_SERVER_TYPES_ALARM_CENTER,  ///< alarm center

SDK_NET_SERVER_TYPES_NETPLAT_MEGA,

SDK_NET_SERVER_TYPES_NETPLAT_XINWANG,

SDK_NET_SERVER_TYPES_NETPLAT_SHISOU,

SDK_NET_SERVER_TYPES_NETPLAT_VVEYE,

SDK_NET_SERVER_TYPES_RTSP,     //RTSP

SDK_NET_SERVER_TYPES_NR,

};

/// preview type

enum SDK_PreviewTypes

{

SDK_PREVIEW_TYPES_TOUR,		///< tour

SDK_PREVIEW_TYPES_TALK,		///< talk

SDK_PREVIEW_TYPES_NR

};

enum SDK_StorageDeviceControlTypes

{

SDK_STORAGE_DEVICE_CONTROL_SETTYPE,		///< set disk type

SDK_STORAGE_DEVICE_CONTROL_RECOVER,		///< recover error

SDK_STORAGE_DEVICE_CONTROL_PARTITIONS,	///< disk partition

SDK_STORAGE_DEVICE_CONTROL_CLEAR,		///< clear data

SDK_STORAGE_DEVICE_CONTROL_NR,

};

/// clear type of disk data

enum SDK_StorageDeviceClearTypes

{

SDK_STORAGE_DEVICE_CLEAR_DATA,			///< clear record data

SDK_STORAGE_DEVICE_CLEAR_PARTITIONS,	///< clear partitions

SDK_STORAGE_DEVICE_CLEAR_NR,

};

/// disk type

enum SDK_FileSystemDriverTypes

{

SDK_DRIVER_READ_WRITE	= 0,

SDK_DRIVER_READ_ONLY	= 1,

SDK_DRIVER_EVENTS		= 2,

SDK_DRIVER_REDUNDANT	= 3,

SDK_DRIVER_SNAPSHOT		= 4,

SDK_DRIVER_TYPE_NR		= 5,

SDK_DRIVER_UNUSED		= 0xff,

};

/// default configuration type

enum SDK_DefaultConfigKinds

{

SDK_DEFAULT_CFG_GENERAL,			// general

SDK_DEFAULT_CFG_ENCODE,				// encode

SDK_DEFAULT_CFG_RECORD,				// record

SDK_DEFAULT_CFG_NET_SERVICE,		// net services

SDK_DEFAULT_CFG_NET_COMMON,			// network

SDK_DEFAULT_CFG_ALARM,				// alarm

SDK_DEFAULT_CFG_PTZCOMM,			// PTZ/232

SDK_DEFAULT_CFG_USERMANAGER,		// user management

SDK_DEFAULT_CFG_PREVIEW,			// preview

SDK_DEFAULT_CFG_END,

};

enum SDK_File_Type

{

SDK_RECORD_ALL = 0,

SDK_RECORD_ALARM = 1, //ALARM RECORD

SDK_RECORD_DETECT,	  //VIDEO DETECT RECORD

SDK_RECORD_REGULAR,	  //REGULAR RECORD

SDK_RECORD_MANUAL,	  //MANUAL RECORD

SDK_PIC_ALL = 10,

SDK_PIC_ALARM,		  //ALARM PICTURE

SDK_PIC_DETECT,		  //VIDEO DETECT PICTURE

SDK_PIC_REGULAR,      //REGULAR PICTURE

SDK_PIC_MANUAL,       //MANUAL PICTURE

SDK_TYPE_NUM

};

//PlayBack Action

enum SEDK_PlayBackAction

{

SDK_PLAY_BACK_PAUSE,		///< Pause

SDK_PLAY_BACK_CONTINUE,		///< Resume

SDK_PLAY_BACK_SEEK,			///< Seek

SDK_PLAY_BACK_FAST,	        ///< Fast

SDK_PLAY_BACK_SLOW,	        ///< slow

SDK_PLAY_BACK_SEEK_PERCENT, /*<! PlayBack Pos */

};

### Structure of device information

- Structure of device define as below

typedef struct _H264_DVR_DEVICEINFO

{

char sSoftWareVersion[64];	///< software version

char sHardWareVersion[64];	///< hardware version

char sEncryptVersion[64];	///< encrypt version

SDK_SYSTEM_TIME tmBuildTime;///< build time

char sSerialNumber[64];			///< device serial number

int byChanNum;				///< channel number of video in

int iVideoOutChannel;		///< channel number of video out

int byAlarmInPortNum;		///< channel number of alarm in

int byAlarmOutPortNum;		///< channel number of alarm out

int iTalkInChannel;			///< channel number of talk in

int iTalkOutChannel;		///< channel number of talk out

int iExtraChannel;	         ///< channel number of extra

int iAudioInChannel;		     ///< channel number of audio in

int iCombineSwitch;			///< channel number of combine

int iDigChannel;		///<Digital Channel number

unsigned int uiDeviceRunTime;	///<System Run Time

}H264_DVR_DEVICEINFO,*LPH264_DVR_DEVICEINFO;

### Date Information

typedef struct SDK_SYSTEM_TIME{

int  year;///< year

int  month;///< month，January = 1, February = 2, and so on.

int  day;///< day

int  wday;///< week, Sunday = 0, Monday = 1, and so on

int  hour;///< hour

int  minute;///< minute

int  second;///< second

int  isdst;///< DST(daylight saving time) flag, Yes = 1, No = 0

}SDK_SYSTEM_TIME;

### Record File Information

//search condition structure

typedef struct

{

int nChannelN0;			//channel NO, start with 0

int nFileType;			//record type

H264_DVR_TIME startTime;	//start time

H264_DVR_TIME endTime;	//end time

char szCard[32];			//card number

void *hWnd	//the handle of play window（if NULL,net data will separate from decoding and playing）

}H264_DVR_FINDINFO;

//the return of record information structure

typedef struct

{

int ch;					///< channel NO, start with 0

int size;					///< record size(BYTE)

char sFileName[108];		///< record file name

SDK_SYSTEM_TIME stBeginTime;	///< start time of record	SDK_SYSTEM_TIME stEndTime;		///< end time of record

void *hWnd	//the handle of play window（if NULL,net data will separate from decoding and playing）

}H264_DVR_FILE_DATA;

//Search by Time

struct SDK_SearchByTime

{

int nHighChannel;			///< bitmask for 33~64 channel

int nLowChannel;			///< bitmask for 1~32 channel

int nFileType;              ///< 文件类型, 见SDK_File_Type

SDK_SYSTEM_TIME stBeginTime;	///< begin time

SDK_SYSTEM_TIME stEndTime;		///< end time

int    iSync;               ///< is sync

};

//record info for each channel

struct SDK_SearchByTimeInfo

{

int iChannel;			    ///< record channel

///< 0000:NO Record 0001:F_COMMON 0002:F_ALERT 0003:F_DYNAMIC 0004:F_CARD 0005:F_HAND

unsigned char cRecordBitMap[720];

};

struct SDK_SearchByTimeResult

{

int nInfoNum;										 ///< number of record info

SDK_SearchByTimeInfo ByTimeInfo[NET_MAX_CHANNUM];    ///< record info

};

- Protocol Of Serial Information

struct SDK_COMMATTRI

{

int	iDataBits;	// data bit: [5,8]

int	iStopBits;	// stop bit: [0,2]

int	iParity;	// parity: 0: None 1: odd 2：even 3: mark 4: space

int	iBaudRate;	// baudrate: 115200,57600,38400,9600,4800,2400 and so on

};

// serial configure

struct SDK_CONFIG_COMM_X

{

char iProtocolName[32];	// Protocol:“Console”

int iPortNo;		// Port No.

SDK_COMMATTRI aCommAttri;		// attribute of serial

};

- PTZ  Protocol

struct SDK_STR_CONFIG_PTZ

{

char sProtocolName[NET_MAX_PTZ_PROTOCOL_LENGTH];	// Protocol

int	ideviceNo;				// PTZ device NO.

int	iNumberInMatrixs;		// NO. in matrixs

int iPortNo;				// serial port NO.:	[1, 4]

SDK_COMMATTRI dstComm;			// attribute of serial

};

//all channel of PTZ protocol

struct SDK_STR_PTZCONFIG_ALL

{

SDK_STR_CONFIG_PTZ ptzAll[NET_MAX_CHANNUM];

};

- User Manager Data Structure

Right List

typedef struct _OPR_RIGHT

{

string		name;

}OPR_RIGHT;

typedef struct _USER_INFO

{

int			rigthNum;

string		rights[NET_MAX_RIGTH_NUM];

string		strGroupname;

string		strmemo;

string		strname;

string		strpassWord;

bool		reserved;		//is reserved user

bool		shareable;		//is shareable

}USER_INFO;

typedef struct _USER_GROUP_INFO

{

int			rigthNum;

string		memo;

string		name;

string		rights[NET_MAX_RIGTH_NUM];	//right list

}USER_GROUP_INFO;

//all of user and group information structure

typedef struct _USER_MANAGE_INFO

{

int					rigthNum;

OPR_RIGHT			rightList[NET_MAX_RIGTH_NUM];

int					groupNum;

USER_GROUP_INFO		groupList[NET_MAX_GROUP_NUM];

int					userNum;

USER_INFO			userList[NET_MAX_USER_NUM];

}USER_MANAGE_INFO;

//modify user

typedef struct _CONF_MODIFYUSER

{

std::string sUserName;

USER_INFO User;

}CONF_MODIFYUSER;

//modify group

typedef struct _CONF_MODIFYGROUP

{

std::string sGroupName;

USER_GROUP_INFO Group;

}CONF_MODIFYGROUP;

/// modify password

struct _CONF_MODIFY_PSW

{

std::string sUserName;

std::string sPassword;

std::string sNewPassword;

};

- Log Information

#define NET_MAX_RETURNED_LOGLIST	1024		//the max item of Log

/// log search condition

struct SDK_LogSearchCondition

{

int nType;	///< log type: 0: all 1: system 2: configure 3: storage 4: alarm 5：record 6: account 7: file

int iLogPosition;			///< return of last log item position in the whole logs

SDK_SYSTEM_TIME stBeginTime;	///< begin time

SDK_SYSTEM_TIME stEndTime;		///< end time

};

//return of Log search

struct SDK_LogList

{

int iNumLog;

struct LogItem

{

char sType[24];	///< log type

char sUser[32];	///< Operator of log

char sData[68];	///< log data

SDK_SYSTEM_TIME stLogTime;	///< the time of log happened

} Logs[NET_MAX_RETURNED_LOGLIST];

};

- Storage information structure

struct SDK_STORAGEDISK

{

int		iPhysicalNo;

int		iPartNumber;

SDK_DriverInformation diPartitions[SDK_MAX_DRIVER_PER_DISK];

};

struct SDK_StorageDeviceInformationAll

{

int iDiskNumber;

SDK_STORAGEDISK vStorageDeviceInfoAll[SDK_MAX_DISK_PER_MACHINE];

};

- Real-Time Monitor

typedef struct{

int nChannel;	//Channel NO.

int nStream;	//0: main stream，1: extra stream

int nMode;	//0：TCP, 1：UDP

int nComType;	//only efficient for combination encode，for jigsaw module

void* hWnd;	//the handle of preview window（if NULL,net data will separate from decoding and playing）

}H264_DVR_CLIENTINFO,*LPH264_DVR_CLIENTINFO;

### Structure of Configuration

Commands of H264_DVR_GetDevConfig、H264_DVR_SetDevConfig

typedef enum _SDK_CONFIG_TYPE

{

E_SDK_CONFIG_NOTHING = 0,		//

E_SDK_CONFIG_USER,			//User information，including power list，user list and group list

E_SDK_CONFIG_ADD_USER,		//add user

E_SDK_CONFIG_MODIFY_USER,	//modify user

E_SDK_CONFIG_DELETE_USER,   //del user

E_SDK_CONFIG_ADD_GROUP,		//add group

E_SDK_CONFIG_MODIFY_GROUP,	//modify group

E_SDK_COFIG_DELETE_GROUP,   //del group

E_SDK_CONFIG_MODIFY_PSW,	//modify password

E_SDK_CONFIG_ABILITY_SYSFUNC = 9,//support network services

E_SDK_CONFIG_ABILTY_ENCODE,	//encode ability

E_SDK_CONFIG_ABILITY_PTZPRO,	//protocols of ptz support

E_SDK_COMFIG_ABILITY_COMMPRO,	//protocols of 232 support

E_SDK_CONFIG_ABILITY_MOTION_FUNC,	//Motion detect

E_SDK_CONFIG_ABILITY_BLIND_FUNC,	//Blind detect

E_SDK_CONFIG_ABILITY_DDNS_SERVER,	//type  of DDNS services support

E_SDK_CONFIG_ABILITY_TALK,		//encode type of audio talk support

E_SDK_CONFIG_SYSINFO = 17,		//system information

E_SDK_CONFIG_SYSNORMAL,	//general

E_SDK_CONFIG_SYSENCODE,	//encode

E_SDK_CONFIG_SYSNET,		//network

E_SDK_CONFIG_PTZ,			//ptz

E_SDK_CONFIG_COMM,		//232

E_SDK_CONFIG_RECORD,		//record

E_SDK_CONFIG_MOTION,		//motion detect

E_SDK_CONFIG_SHELTER,		//blind detect

E_SDK_CONFIG_VIDEO_LOSS,  //loss detect

E_SDK_CONFIG_ALARM_IN,	//alarm in

E_SDK_CONFIG_ALARM_OUT,	//alarm out

E_SDK_CONFIG_DISK_MANAGER,//disk management

E_SDK_CONFIG_OUT_MODE,	//out mode

E_SDK_CONFIG_CHANNEL_NAME,//channel name

E_SDK_CONFIG_AUTO,		//auto maintain

E_SDK_CONFIG_DEFAULT,     //set default

E_SDK_CONFIG_DISK_INFO,	//disk info

E_SDK_CONFIG_LOG_INFO,	//get log

E_SDK_CONFIG_NET_IPFILTER, //network services：black/white list

E_SDK_CONFIG_NET_DHCP, //network services：DHCP

E_SDK_CONFIG_NET_DDNS, //network services：DDNS

E_SDK_CONFIG_NET_EMAIL, //network services：EMAIL

E_SDK_CONFIG_NET_MULTICAST, //network services:Multicast

E_SDK_CONFIG_NET_NTP, //network services： NTP

E_SDK_CONFIG_NET_PPPOE, //network services:  PPPOE

E_SDK_CONFIG_NET_DNS, //network services:  DNS

E_SDK_CONFIG_NET_FTPSERVER, //network services: FTP

E_SDK_CONFIG_SYS_TIME,	//system time

E_SDK_CONFIG_CLEAR_LOG,	//clear log

E_SDK_REBOOT_DEV,		//reboot device

E_SDK_CONFIG_ABILITY_LANG,	//languages support

E_SDK_CONFIG_VIDEO_FORMAT,  // video format

E_SDK_CONFIG_COMBINEENCODE,	//combine-encode

E_SDK_CONFIG_EXPORT,	//config export

E_SDK_CONFIG_IMPORT,	//config import

E_SDK_LOG_EXPORT,		//log export

E_SDK_CONFIG_COMBINEENCODEMODE, //mode of combine-encode

E_SDK_WORK_STATE,	//work status

E_SDK_ABILITY_LANGLIST, //language list

E_SDK_CONFIG_NET_ARSP,  // ARSP

E_SDK_CONFIG_SNAP_STORAGE, // Snapshot record

E_SDK_CONFIG_NET_3G, //3G dial

E_SDK_CONFIG_NET_MOBILE, //mobile monitor

E_SDK_CONFIG_UPGRADEINFO, //get upgrade info

E_SDK_CONFIG_NET_DECODER,

E_SDK_ABILITY_VSTD,

E_SDK_CONFIG_ABILITY_VSTD,

E_SDK_CONFIG_NET_UPNP, //UPUN CFG

E_SDK_CONFIG_NET_WIFI, //WIFI CFG

E_SDK_CONFIG_NET_WIFI_AP_LIST, //WIFI AP LIST

E_SDK_CONFIG_SYSENCODE_SIMPLIIFY, //simplify encode cfg

E_SDK_CONFIG_ALARM_CENTER,  //alarm center config

E_SDK_CONFIG_NET_ALARM,     //net alarm config

E_SDK_CONFIG_NET_MEGA,     //MEGA

E_SDK_CONFIG_NET_XINGWANG, //XINGWANG

E_SDK_CONFIG_NET_SHISOU,   //SHISOU

E_SDK_CONFIG_NET_VVEYE,    //VVEYE

E_SDK_VIDEO_PREVIEW,

E_SDK_CONFIG_NET_DECODER_V2,

E_SDK_CONFIG_ABILITY_SERIALNO,	// Serial ID

E_SDK_CONFIG_NET_DECODER_V3,//Digital Channel

E_SDK_CONFIG_NET_RTSP,    //RTSP

E_SDK_GUISET,              //GUISET

E_SDK_CATCHPIC,               //Catch Picture

E_SDK_CONFIG_OEM_INFO,     //OE

E_SDK_VIDEOCOLOR             //Video Color

E_SDK_CONFIG_COMM485,

E_SDK_COMFIG_ABILITY_COMMPRO485, //RS485

E_SDK_CONFIG_SYS_TIME_NORTC,	//System Time noRtc

E_SDK_CONFIG_REMOTECHANNEL,   //Remote Channel

E_SDK_CONFIG_OPENTRANSCOMCHANNEL , //Open TransComm

E_SDK_CONFIG_CLOSETRANSCOMCHANNEL,  //Close TransComm

E_SDK_CONFIG_SERIALWIRTE,  //Write TransComm Info

E_SDK_CONFIG_SERIALREAD,   //Read TransComm Info

E_SDK_CONFIG_CHANNELTILE_DOT	//Dot

E_SDK_CONFIG_CAMERA,           //Camera Parameter

E_SDK_CONFIG_ABILITY_CAMERA    //the Ability of Exposure

}SDK_CONFIG_TYPE;

/// type of DDNS support

struct SDK_DDNSServiceFunction

{

int  nTypeNum;

char vDDNSType[NET_MAX_DDNS_TYPE][64];

};

/// blind detect support

struct SDK_BlindDetectFunction

{

int iBlindCoverNum;	///< the number of cover area support

};

/// motion detect

struct SDK_MotionDetectFunction

{

int iGridRow;  ///< the number of row

int iGridColumn; ///< the number of colum

};

/// protocols of 232 support

struct SDK_COMMFUNC

{

int nProNum;      ///< the numbers of protocol

char vCommProtocol[SDK_COM_TYPES][32]; ///< the name of protocol

};

/// protocols of PTZ

struct SDK_PTZPROTOCOLFUNC

{

int nProNum;

char vPTZProtocol[100][NET_MAX_PTZ_PROTOCOL_LENGTH];

};

/// encode information

struct SDK_EncodeInfo

{

bool bEnable;			///< enable

int iStreamType;		///< stream type see refer to capture_channel_t

bool bHaveAudio;		///< is support audio

unsigned int uiCompression;		///< mask of capture_comp_t

unsigned int uiResolution;		///< mask of capture_size_t

};

/// encode power

struct CONFIG_EncodeAbility

{

unsigned int iMaxEncodePower;		///< max encode power

int iChannelMaxSetSync;		///< is resolution need sync each channel, 0-no, 1-yes

unsigned int nMaxPowerPerChannel[NET_MAX_CHANNUM];		///< MaxPower Per Channel for main stream

unsigned int ImageSizePerChannel[NET_MAX_CHANNUM];		///< Image Size Per Channel for main stream

unsigned int ExImageSizePerChannel[NET_MAX_CHANNUM];		///< Image Size Per Channel for extern stream

SDK_EncodeInfo vEncodeInfo[SDK_CHL_FUNCTION_NUM];	///< encode information

SDK_EncodeInfo vCombEncInfo[SDK_CHL_FUNCTION_NUM];	///< combine-encode information

int	iMaxBps;				///< MAX BPS Kbps

unsigned int ExImageSizePerChannelEx[NET_MAX_CHANNUM][NET_CAPTURE_SIZE_NUM];	///< Image Size support for each solution

};

///system function

struct SDK_SystemFunction

{

bool vEncodeFunction[SDK_ENCODE_FUNCTION_TYPE_NR];	///< Encode Functions

bool vAlarmFunction[SDK_ALARM_FUNCTION_TYPE_NR];	///< Alarm Fucntions

bool vNetServerFunction[SDK_NET_SERVER_TYPES_NR];	///< Net Server Functions

bool vPreviewFunction[SDK_PREVIEW_TYPES_NR];		///< Preview Functions

};

///< Auto-Maintain setting

struct SDK_AutoMaintainConfig

{

int iAutoRebootDay;			///< interval of Auto-Reboot days

int iAutoRebootHour;			///< time to reboot [0, 23]

int iAutoDeleteFilesDays;		///< interval of Auto-Del record file [0, 30]

};

//Disk info

struct SDK_STORAGEDISK

{

int		iPhysicalNo;       // Physical No.

int		iPartNumber;		// Partition numbers

SDK_DriverInformation diPartitions[SDK_MAX_DRIVER_PER_DISK];

};

struct SDK_StorageDeviceInformationAll

{

int iDiskNumber;

SDK_STORAGEDISK vStorageDeviceInfoAll[SDK_MAX_DISK_PER_MACHINE];

};

// Type of PTZ link

enum PtzLinkTypes

{

PTZ_LINK_NONE,		// NONE

PTZ_LINK_PRESET,		// GOTO PRESET

PTZ_LINK_TOUR,		// TOUR

PTZ_LINK_PATTERN		// PATTERN

};

// PTZ Link Config

struct SDK_PtzLinkConfig

{

int iType;		// see refer to PtzLinkTypes

int iValue;		// value of link type

};

#define CHANNELNAME_MAX_LEN 64  //Length of Channel Name

// handler of event

struct SDK_EventHandler

{

unsigned int	dwRecord;				// bitmask of record. Bit per channel

int	iRecordLatch;			// record latch：10～300 sec.

unsigned int	dwTour;					// bitmask of tour. Bit per channel

unsigned int	dwSnapShot;				// bitmask of snapshot. Bit per channel

unsigned int	dwAlarmOut;				// bitmask of alarm out. Bit per channel

unsigned int	dwMatrix;				// bitmask of matrix. Bit per channel

int		iEventLatch;			// interval of event(unit:sec.)

int		iAOLatch;				// Alarm out latch：10～300 sec

SDK_PtzLinkConfig PtzLink[NET_MAX_CHANNUM];		// PTZ link activation

SDK_CONFIG_WORKSHEET schedule;		// weeksheet of record

bool	bRecordEn;				// enable flag of record

bool	bTourEn;				// enable flag of tour

bool	bSnapEn;				// enable flag of snapshot

bool	bAlarmOutEn;			// enable flag of alarm out

bool	bPtzEn;				// enable flag of PTZ link

bool	bTip;				// enable flag of screen tip

bool	bMail;				// enable flag of sending email

bool	bMessage;				// enable flag of sending message to alarm center

bool	bBeep;					// enable flag of buzzer beep

bool	bVoice;					// enable flag of voice tip

bool	bFTP;					// enable flag of FTP unload

bool	bMatrixEn;				// no used

bool	bLog;					// enable flag of log

bool	bMessagetoNet;			// no used

bool    bShowInfo;              // Show Alarm Information or not

unsigned int    dwShowInfoMask;         //show Channel Mask

char    pAlarmInfo[CHANNELNAME_MAX_LEN];// Alarm Information

};

///< Motion detect

struct SDK_MOTIONCONFIG

{

bool bEnable;							// enable

int iLevel;								// sensitivity：[1,6]

unsigned int mRegion[NET_MD_REGION_ROW];	// regions of motion detect，one bit per column, Max region: 18*22

SDK_EventHandler hEvent;					// handler of motion detect

};

/// All channel of video motion configuration

struct SDK_MOTIONCONFIG_ALL

{

SDK_MOTIONCONFIG vMotionDetectAll[NET_MAX_CHANNUM];

};

///<  Blind detect

struct SDK_BLINDDETECTCONFIG

{

bool	bEnable;		///< enable

int		iLevel;			///< sensitivity：1～6

SDK_EventHandler hEvent;	///< handler of blind detect event

};

/// All channel of blind detect configuration

struct SDK_BLINDDETECTCONFIG_ALL

{

SDK_BLINDDETECTCONFIG vBlindDetectAll[NET_MAX_CHANNUM];

};

///< loss detect

struct SDK_VIDEOLOSSCONFIG

{

bool bEnable;			///< enable

SDK_EventHandler hEvent;	///< handler of loss detect event

};

/// All channel of video loss configuration

struct SDK_VIDEOLOSSCONFIG_ALL

{

SDK_VIDEOLOSSCONFIG vGenericEventConfig[NET_MAX_CHANNUM];

};

///< Alarm in

struct SDK_ALARM_INPUTCONFIG

{

bool	bEnable;		///< enable

int		iSensorType;	///< Sensor Type: Normal Open or Normal Close

SDK_EventHandler hEvent;	///< handler of alarm in

};

///< All channel of alarm in configuration

struct SDK_ALARM_INPUTCONFIG_ALL

{

SDK_ALARM_INPUTCONFIG vAlarmConfigAll[NET_MAX_CHANNUM];

};

///< netalarm config

struct SDK_NETALARMCONFIG

{

bool bEnable;			///< enable

SDK_EventHandler hEvent;	///< event handle

};

/// All channel of netalarm

struct SDK_NETALARMCONFIG_ALL

{

SDK_NETALARMCONFIG vNetAlarmConfig[NET_MAX_CHANNUM];

};

///< Local alarm out

struct SDK_AlarmOutConfig

{

int nAlarmOutType;		///< alarm type: 0:auto 1: manual 2: close

int nAlarmOutStatus;    ///< status: 0:open 1;close

};

///< All channel of Local alarm out

struct SDK_AlarmOutConfigAll

{

SDK_AlarmOutConfig vAlarmOutConfigAll[NET_MAX_CHANNUM];

};

//Vesion:V2

struct SDK_AbilitySerialNo

{

char serialNo[NET_MAX_SERIALNO_LENGTH];

char productType[NET_MAX_SERIALNO_LENGTH];

};

///< Driver information

struct SDK_DriverInformation

{

int		iDriverType;		///< driver type

bool	bIsCurrent;			///< is work disk

unsigned int	uiTotalSpace;		///< total space,(unit:MB)

unsigned int	uiRemainSpace;		///< remain space,(unit:MB)

int		iStatus;			///< status

int		iLogicSerialNo;				///< logic No.

SDK_SYSTEM_TIME  tmStartTimeNew;		///< new record start time

SDK_SYSTEM_TIME	 tmEndTimeNew;			///< new record end time

SDK_SYSTEM_TIME	 tmStartTimeOld;		///< old record start time

SDK_SYSTEM_TIME	 tmEndTimeOld;			///< old record end time

};

/// record mode type

enum SDK_RecordModeTypes

{

SDK_RECORD_MODE_CLOSED,		///< Closed

SDK_RECORD_MODE_MANUAL,		///< Manual: record all the time

SDK_RECORD_MODE_CONFIG,		///< Configuration: according to SDK_RECORDCONFIG

SDK_RECORD_MODE_NR,

};

///< record setting

struct SDK_RECORDCONFIG

{

int iPreRecord;			 ///< pre-record time（Unit:sec.）

bool bRedundancy;		     ///< redundancy record

bool bSnapShot;			///< no used

int iPacketLength;		    ///< record length（unit:minute）[1, 255]

int iRecordMode;		    ///< record mode，refer to SDK_RecordModeTypes

SDK_CONFIG_WORKSHEET wcWorkSheet;			///< worksheet

unsigned int typeMask[NET_N_WEEKS][NET_N_TSECT];		///< mask of record type, corresponding to worksheet

};

// All channel of record configuration

struct SDK_RECORDCONFIG_ALL

{

SDK_RECORDCONFIG vRecordConfigAll[NET_MAX_CHANNUM];

};

// General Configuration

typedef struct _SDK_CONFIG_NORMAL

{

SDK_SYSTEM_TIME sysTime;		 ///< system time

int iLocalNo;			     ///< device No.:[0, 998]

int iOverWrite;			///< when disk full, 0: OverWrite, 1: StopRecord

int iSnapInterval;			///< no used

char sMachineName[64];	    ///< device name

int iVideoStartOutPut;	    ///< no used

int iAutoLogout;		    ///< auto logout [0, 120], 0 means never

int iVideoFormat;		    ///< video format: 0:PAL, 1:NTSC, 2:SECAM

int iLanguage;			///< language: 0:English, 1: SimpChinese, 2:TradChinese, 3: Italian, 4:Spanish, 5:Japanese, 6:Russian, 7:French, 8:German

int iDateFormat;		///< date format: 0:YYMMDD, 1:MMDDYY, 2:DDMMYY

int iDateSeparator;		///< Date separator: 0: ., 1: -, 2: /

int iTimeFormat;		///< Time format: 0: 12, 1: 24

int iDSTRule;			///< DST rule: 0: OFF, 1: ON

int iWorkDay;			///< work day

DSTPoint dDSTStart;

DSTPoint dDSTEnd;

}SDK_CONFIG_NORMAL;

// encode configuration

struct SDK_CONFIG_ENCODE

{

SDK_MEDIA_FORMAT dstMainFmt[SDK_ENCODE_TYPE_NUM];		//  main stream

SDK_MEDIA_FORMAT dstExtraFmt[SDK_EXTRATYPES];	         //  Extra stream

SDK_MEDIA_FORMAT dstSnapFmt[SDK_ENCODE_TYPE_NUM];		//  Snapshot

};

// all of channel encode configuration

struct  SDK_EncodeConfigAll

{

SDK_CONFIG_ENCODE vEncodeConfigAll[NET_MAX_CHANNUM];

};

/// simplify encode configuration

struct SDK_MEDIA_FORMAT_SIMPLIIFY

{

SDK_VIDEO_FORMAT vfFormat;		//  media format

bool	bVideoEnable;			//  video enable flag

bool	bAudioEnable;			//  audio enable flag

};

/// Encode Set

struct SDK_CONFIG_ENCODE_SIMPLIIFY

{

SDK_MEDIA_FORMAT dstMainFmt;	//  main stream

SDK_MEDIA_FORMAT dstExtraFmt;	//  Extra stream

};

struct SDK_EncodeConfigAll_SIMPLIIFY

{

SDK_CONFIG_ENCODE_SIMPLIIFY vEncodeConfigAll[NET_MAX_CHANNUM];

};

// combine-encode

struct  SDK_CombineEncodeConfigAll

{

SDK_CONFIG_ENCODE vEncodeConfigAll[NET_MAX_COMBINE_NUM];

};

/// combine-encode mode

struct SDK_CombEncodeParam

{

int iEncodeMode;	 //refer to CombineEncodeMode

};

struct SDK_CombEncodeModeAll

{

SDK_CombEncodeParam vEncodeParam[NET_MAX_COMBINE_NUM];

};

// video widget

struct  SDK_VIDEO_WIDGET

{

unsigned int rgbaFrontground;		///< front ground color

unsigned int rgbaBackground;		///< back ground color

sdkRect	rcRelativePos;			///< show rect (Total: 8191*8191)

bool	bPreviewBlend;			///< preview overlap

bool	bEncodeBlend;			///< encode overlap

};

// video widget configuration

struct SDK_CONFIG_VIDEOWIDGET

{

SDK_VIDEO_WIDGET	dstCovers[NET_COVERNUM];

SDK_VIDEO_WIDGET	ChannelTitle;

SDK_VIDEO_WIDGET	TimeTitle;

struct

{

char strName[NET_NAME_PASSWORD_LEN];

__int64 iSerialNo;

}ChannelName;					///< channel name

int				iCoverNum;		///< numbers of cover region

};

// all of channel video widget

struct SDK_VideoWidgetConfigAll

{

SDK_CONFIG_VIDEOWIDGET vVideoWidegetConfigAll[NET_MAX_CHANNUM];

};

//Video Color

struct SDK_VIDEOCOLOR_PARAM

{

int	nBrightness;		///< Bright	0-100

int	nContrast;			///< Contrast	0-100

int	nSaturation;		///< Saturation	0-100

int	nHue;				///< Hue	0-100

int	mGain;				///< Gain	0-100 In bit7,"1" is Auto Gain

int	mWhitebalance;		///< Whitebalance，in bit7,"1" is auto control .0x0,0x1,0x2 is low ,mid, high

int nAcutance;          ///< Acutance   0-15

};

///< Video Color Struct

struct SDK_VIDEOCOLOR

{

SDK_TIMESECTION			tsTimeSection;		/// Time Section

SDK_VIDEOCOLOR_PARAM	dstColor;			/// Color struct

int					iEnable;

};

struct SDK_CONFIG_VIDEOCOLOR

{

SDK_VIDEOCOLOR dstVideoColor[NET_N_MIN_TSECT];

};

struct SDK_VideoColorConfigAll

{

SDK_CONFIG_VIDEOCOLOR vVideoColorAll[NET_MAX_CHANNUM];

};

struct SDK_ChannelNameConfigAll

{

char channelTitle[NET_MAX_CHANNUM][NET_NAME_PASSWORD_LEN];

};

//OutMode

struct SDK_GUISetConfig

{

int iWindowAlpha;			///< [128, 255]

bool bTimeTitleEn;			///Time title

bool bChannelTitleEn;		///< Channel Title

bool bAlarmStatus;			///<  Alarm Status

bool bRecordStatus;			///<  Record Status

bool bChanStateRecEn;		///<  Record Sign

bool bChanStateVlsEn;		///<  Video Loss

bool bChanStateLckEn;		///< Channel Lock

bool bChanStateMtdEn;		///< Motion

bool bBitRateEn;			///< BitRate

bool bRemoteEnable;			///< RemoteControl

bool bDeflick;

};

// newwork configuration

struct SDK_CONFIG_NET_COMMON

{

char HostName[NET_NAME_PASSWORD_LEN]; ///< hostname

CONFIG_IPAddress HostIP;   			///< IP

CONFIG_IPAddress Submask; 			///< Netmask

CONFIG_IPAddress Gateway;			///< NetGateway

int HttpPort;						///< HTTP port

int TCPPort;						///< TCP port

int SSLPort;						///< no used

int UDPPort;						///< no used

int MaxConn;						///< max connect

int MonMode;						///< translation protocol: 0:TCP, 1: UDP, 2: MCAST, only support TCP now

int MaxBps;					///< no used

int TransferPlan;				///< Translation policy: 0: AUTO 1: Quality first 2: fluency first

bool bUseHSDownLoad;			///< flag of high speed download

char sMac[NET_MAX_MAC_LEN];     //!MAC addr

unsigned int DeviceID; ///< Device ID

};

//DHCP

struct SDK_NetDHCPConfig

{

bool bEnable;

char ifName[32];

};

struct SDK_NetDHCPConfigAll

{

SDK_NetDHCPConfig vNetDHCPConfig[2];

};

///< DNS

struct SDK_NetDNSConfig

{

CONFIG_IPAddress		PrimaryDNS;

CONFIG_IPAddress		SecondaryDNS;

};

///< net server structure

struct SDK_RemoteServerConfig

{

char ServerName[NET_NAME_PASSWORD_LEN];	///< server name

CONFIG_IPAddress ip;						///< IP

int Port;							///< port

char UserName[NET_NAME_PASSWORD_LEN];		///< user name

char Password[NET_NAME_PASSWORD_LEN];		///< password

bool Anonymity;							///< is Anonymity, for FTP

};

// PTZ configuration

struct SDK_STR_CONFIG_PTZ

{

char sProtocolName[NET_MAX_PTZ_PROTOCOL_LENGTH];	//< Protocol name

int	ideviceNo;			//< device No.

int	iNumberInMatrixs;		//< No. in Matrixs

int iPortNo;				//< Port No.[1, 4]

SDK_COMMATTRI dstComm;		//< comm attribute

};

// all channel of PTZ configuration

struct SDK_STR_PTZCONFIG_ALL

{

SDK_STR_CONFIG_PTZ ptzAll[NET_MAX_CHANNUM];

};

// 232 configuration

struct SDK_CONFIG_COMM_X

{

char iProtocolName[32];	// Protocol name:“Console”

int iPortNo;		// Port No.

SDK_COMMATTRI aCommAttri;		// comm attribute

};

// all channel of 232 configuration

struct SDK_CommConfigAll

{

SDK_CONFIG_COMM_X vCommConfig[SDK_COM_TYPES];

};

//RS485

struct SDK_STR_RS485CONFIG_ALL

{

SDK_STR_CONFIG_PTZ ptzAll[NET_MAX_CHANNUM];

};

struct SDK_CONFIG_WORKSHEET

{

SDK_TIMESECTION	tsSchedule[NET_N_WEEKS][NET_N_TSECT];	///< worksheet

};

///< IP Fliter

struct SDK_NetIPFilterConfig

{

bool Enable;		///< enable

CONFIG_IPAddress BannedList[NET_MAX_FILTERIP_NUM];		///< black list

CONFIG_IPAddress TrustList[NET_MAX_FILTERIP_NUM];		///< white list

};

///< multicast

struct SDK_NetMultiCastConfig

{

bool Enable;		///< enable

SDK_RemoteServerConfig Server;		///< multicast server

};

///< pppoe

struct SDK_NetPPPoEConfig

{

bool Enable;	///< enable

SDK_RemoteServerConfig Server;		///< PPPOE server

CONFIG_IPAddress addr;		///< ip get from PPPOE dial

};

///< DDNS

struct SDK_NetDDNSConfig

{

bool Enable;	///< enable

bool Online;		///<Online or not

char DDNSKey[NET_NAME_PASSWORD_LEN];	///< the type of DDNS name

char HostName[NET_NAME_PASSWORD_LEN];	///< hostname

SDK_RemoteServerConfig Server;		///< DDNS server

};

///< DDNS

struct SDK_NetDDNSConfigALL

{

SDK_NetDDNSConfig ddnsConfig[5];

};

///< FTP

struct SDK_FtpServerConfig {

///< enable

bool Enable;

///< FTP server

SDK_RemoteServerConfig Server;

///< spare server IP

CONFIG_IPAddress SpareIP;

///< path name in FTP server

char RemotePathName[NET_MAX_PATH_LENGTH];

///< max file lenght

int  FileMaxLen;

///< upload periods

SDK_TIMESECTION  UpLoadPeriod[NET_N_MIN_TSECT];

};

///< NTP

struct SDK_NetNTPConfig

{

///< enable

bool Enable;

///< NTP server

SDK_RemoteServerConfig Server;

///< update period

int UpdatePeriod;

///< time zone

int TimeZone;

};

#define  NET_MAX_EMAIL_TITLE_LEN 64

#define  NET_MAX_EMAIL_RECIEVERS  5

#define  NET_EMAIL_ADDR_LEN  32

///< EMAIL

struct SDK_NetEmailConfig

{

///< enalbe

bool Enable;

///< smtp server

SDK_RemoteServerConfig Server;

///< is need SSL ?

bool bUseSSL;

///< sender address

char SendAddr[NET_EMAIL_ADDR_LEN];

///< receiver

char Recievers[NET_MAX_EMAIL_RECIEVERS][NET_EMAIL_ADDR_LEN];

///< email title

char Title[NET_MAX_EMAIL_TITLE_LEN];

///< time section

SDK_TIMESECTION Schedule[NET_N_MIN_TSECT];

};

///< ARSP

struct SDK_NetARSPConfig

{

bool bEnable;	///< enable

char sARSPKey[NET_NAME_PASSWORD_LEN];	///< ARSP name

int iInterval;	///< update period

char sURL[NET_NAME_PASSWORD_LEN];	///< ARSP server URL

SDK_RemoteServerConfig Server;		///< ARSP server

int nHttpPort;   // ARSP server HTTP port

};

struct SDK_NetARSPConfigAll

{

SDK_NetARSPConfig vNetARSPConfigAll[NET_MAX_ARSP_TYPE];

};

struct SDK_NetDecoderConfig

{

bool Enable;						///< Enable

char UserName[NET_NAME_PASSWORD_LEN];	///< DDNS Type, Current: YOUTANG

char PassWord[NET_NAME_PASSWORD_LEN];	///< Hostname

char Address[NET_NAME_PASSWORD_LEN];

int Protocol;

int Port;							///< Port

int Channel;						///< Channel num

int Interval;                       ///< time(s)

};

struct SDK_NetDecoderConfigAll

{

SDK_NetDecoderConfig vNetDecoderConfig[NET_MAX_DECORDR_CH];

};

//Vesion:V2

struct SDK_NetDecoderConfig_V2

{

int nTotalNum;            //MaxNum:NET_MAX_CHANNUM

SDK_NetDecoderConfig vNetDecoderConfig[NET_MAX_CHANNUM];

};

//Vesion:V2

struct SDK_NetDecoderConfigAll_V2

{

SDK_NetDecoderConfig_V2 vNetDecoderArray[NET_MAX_DECORDR_CH];

};

///< Decord Config

struct SDK_NetDecorderConfigV3

{

bool Enable;						///< Enable

char UserName[NET_NAME_PASSWORD_LEN];	///< DDNS Type, Current: YOUTANG

char PassWord[NET_NAME_PASSWORD_LEN];	///< Hostname

char Address[NET_NAME_PASSWORD_LEN];

int Protocol;

int Port;							///< Port

int Channel;						///< Channel num

int Interval;                       ///< Interval time

char ConfName[NET_NAME_PASSWORD_LEN];	///<Config Name

int DevType;						///<Device Type

int StreamType;						///<Frame Type based on CaptureChannelTypes

};

/*Link Type*/

enum SDK_DecorderConnType

{

SDK_CONN_SINGLE = 0, 	//Single link

SDK_CONN_MULTI = 1,		/*Multi link*/

SDK_CONN_TYPE_NR,

};

/*Digital Channel Config*/

struct SDK_NetDigitChnConfig

{

bool Enable;		/*Enble*/

int ConnType;		/*Link Type*/

int TourIntv;		/*Tour Interval*/

unsigned int SingleConnId;	/*Single Link, 1:Start，0:error*/

bool EnCheckTime;	/*Check Time*/

SDK_NetDecorderConfigV3 NetDecorderConf[32]; /*Config table*/

};

/*All Digital Channel Config*/

struct SDK_NetDecorderConfigAll_V3

{

SDK_NetDigitChnConfig DigitChnConf[NET_MAX_DECORDR_CH];

};

///< DNS

struct SDK_NetDNSConfig

{

CONFIG_IPAddress		PrimaryDNS;

CONFIG_IPAddress		SecondaryDNS;

};

/// audio format for audio talk

struct SDK_AudioInFormatConfig

{

int iBitRate;	   ///< bitrate，(unit:kbps)

int iSampleRate;   ///< sample rate(unit:Hz)

int iSampleBit;    ///< sample bit

int iEncodeType;   ///< type of encode，see refer to AudioEncodeTypes

};

/// alarm status

struct SDK_DVR_ALARMSTATE

{

int iVideoMotion; ///< motion detect status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iVideoBlind; ///< blind detect status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iVideoLoss;	///< loss detect status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iAlarmIn;	///< alarm in status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iAlarmOut;	///< alarm out status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

};

// channel status

struct SDK_DVR_CHANNELSTATE

{

bool bRecord; ///< is recording

int iBitrate;	///< bitrate

};

// device work status

struct SDK_DVR_WORKSTATE

{

SDK_DVR_CHANNELSTATE vChnState[NET_MAX_CHANNUM];

SDK_DVR_ALARMSTATE vAlarmState;

};

/// language support

struct SDK_MultiLangFunction

{

///< the numbers of language

int nLangNum;

///< language name

char vLanguageName[128][64];

};

/// type of default configuration

struct SDK_SetDefaultConfigTypes

{

bool vSetDefaultKinds[SDK_DEFAULT_CFG_END];

};

/// audio talk format

struct SDK_AudioInFormatConfigAll

{

SDK_AudioInFormatConfig vAudioInFormatConfig[SDK_AUDIO_ENCODE_TYPES_NR];

};

// upgrade information

struct SDK_UpgradeInfo

{

char szSerial[64];

char szHardware[64];

char szVendor[64];

unsigned int uiLogoArea[2]; //the flash zone of LOGO

};

typedef struct {

int left;

int top;

int right;

int bottom;

}sdkRect;

/// audio format

struct SDK_AudioInFormatConfig

{

int iBitRate;	   ///< bitrate，(unit:kbps), such as 192kbps，128kbps

int iSampleRate;   ///< sample rate(unit:Hz), such as 44100Hz

int iSampleBit;    ///< sample bit

int iEncodeType;   ///< type of encode，see refer to AudioEncodeTypes

};

// audio talk encode type

typedef enum __TALK_CODING_TYPE

{

TALK_DEFAULT = 0,

TALK_PCM = 1,					//PCM

TALK_G711a,						//G711a

TALK_AMR,						//AMR

TALK_G711u,						//G711u

TALK_G726,						//G726

}TALK_CODING_TYPE;

typedef struct

{

TALK_CODING_TYPE		encodeType;		//encode type

int						nAudioBit;		//audio sample bit

unsigned int			dwSampleRate;	//sample rate

char					reserved[64];

}H264_DVR_TALKDECODE_INFO;

/// Video format support

struct SDK_MultiVstd

{

int nVstdNum;

char vVstdName[3][64];

};

struct SDK_VIDEO_FORMAT

{

int		iCompression;			//  refer to SDK_CAPTURE_COMP_t

int		iResolution;			//  refer to capture_size_t

int		iBitRateControl;		//  refer to capture_brc_t(DVRAPI.H)

int		iQuality;				//  quality lever[1, 6]

int		nFPS;					//  frame per second, PAL:[1, 25] NTFC:[1, 30]

int		nBitRate;				//  bitrate

int		iGOP;					//  interval of 2 key frame [2,12]

} ;

struct  SDK_AUDIO_FORMAT

{

int		nBitRate;				//  bitrate(unit:kbps)

int		nFrequency;				//  sample rate

int		nMaxVolume;				//  max volume

} ;

// 媒体格式

struct SDK_MEDIA_FORMAT

{

SDK_VIDEO_FORMAT vfFormat;			//  refer to SDK_VIDEO_FORMAT

SDK_AUDIO_FORMAT afFormat;			//  refer to SDK_AUDIO_FORMAT

bool	bVideoEnable;				//  enable video decode

bool	bAudioEnable;				//  enable audio decode

} ;

typedef union {	//IP addr

unsigned char	c[4];

unsigned short	s[2];

unsigned int 	l;

}CONFIG_IPAddress;

### Network keyboard define

/// KEY VALUE

enum SDK_NetKeyBoardValue

{

SDK_NET_KEY_0, SDK_NET_KEY_1, SDK_NET_KEY_2, SDK_NET_KEY_3, SDK_NET_KEY_4, SDK_NET_KEY_5, SDK_NET_KEY_6, SDK_NET_KEY_7, SDK_NET_KEY_8, SDK_NET_KEY_9,

SDK_NET_KEY_10, SDK_NET_KEY_11, SDK_NET_KEY_12, SDK_NET_KEY_13, SDK_NET_KEY_14, SDK_NET_KEY_15, SDK_NET_KEY_16, SDK_NET_KEY_10PLUS,

SDK_NET_KEY_UP = 20,     // UP

SDK_NET_KEY_DOWN,        // DOWN

SDK_NET_KEY_LEFT,        // LEFT

SDK_NET_KEY_RIGHT,       // RIGHT

SDK_NET_KEY_SHIFT,

SDK_NET_KEY_PGUP,        // PAGE UP

SDK_NET_KEY_PGDN,        // PAGE DOWN

SDK_NET_KEY_RET,         // ENTER

SDK_NET_KEY_ESC,         // ESC

SDK_NET_KEY_FUNC,        // FUNC

SDK_NET_KEY_PLAY,        // PLAY/PAUSE

SDK_NET_KEY_BACK,        // BACK

SDK_NET_KEY_STOP,        // STOP

SDK_NET_KEY_FAST,        // FAST

SDK_NET_KEY_SLOW,        // SLOW

SDK_NET_KEY_NEXT,        // NEXT FILE

SDK_NET_KEY_PREV,        // PREV FILE

SDK_NET_KEY_REC = 40,    // ENTER RECORD SETTING PAGE

SDK_NET_KEY_SEARCH,      // ENTER RECORD SEARCH PAGE

SDK_NET_KEY_INFO,        // ENTER SYSTEM INFO PAGE

SDK_NET_KEY_ALARM,       // ENTER ALARM OUT PAGE

SDK_NET_KEY_ADDR,        // ENTER REMOTE ADDRESS SETTING PAGE

SDK_NET_KEY_BACKUP,      // ENTER BACKUP PAGE

SDK_NET_KEY_SPLIT,       // NEXT SPLIT MODE

SDK_NET_KEY_SPLIT1,      // SLPIT MODE 1

SDK_NET_KEY_SPLIT4,      // SLPIT MODE 4

SDK_NET_KEY_SPLIT8,      // SLPIT MODE 8

SDK_NET_KEY_SPLIT9,      // SLPIT MODE 9

SDK_NET_KEY_SPLIT16,     // SLPIT MODE 16

SDK_NET_KEY_SHUT,        // SHUTDOWN

SDK_NET_KEY_MENU,        // MENU

SDK_NET_KEY_PTZ = 60,    // ENTER PTZ CONTROL PAGE

SDK_NET_KEY_TELE,        // ZOOM -

SDK_NET_KEY_WIDE,        // ZOOM +

SDK_NET_KEY_IRIS_SMALL,  // APERTURE -

SDK_NET_KEY_IRIS_LARGE,  // APERTURE +

SDK_NET_KEY_FOCUS_NEAR,  // FOCUS -

SDK_NET_KEY_FOCUS_FAR,   // FOCUS +

SDK_NET_KEY_BRUSH,       // BRUSH

SDK_NET_KEY_LIGHT,       // LIGHT

SDK_NET_KEY_SPRESET,     // SET PRESET POINT

SDK_NET_KEY_GPRESET,     // GOTO PRESET POINT

SDK_NET_KEY_DPRESET,     // CLEAR PRESET POINT

SDK_NET_KEY_PATTERN,     // PATTERN

SDK_NET_KEY_AUTOSCAN,    // AUTO-SCAN ON/OFF

SDK_NET_KEY_AUTOTOUR,    // AUTO-TOUR ON/OFF

SDK_NET_KEY_AUTOPAN,     // AUTO-PAN ON/OFF

};

/// keyboard status

enum SDK_NetKeyBoardState

{

SDK_NET_KEYBOARD_KEYDOWN,	// key down

SDK_NET_KEYBOARD_KEYUP,		// key up

};

struct SDK_NetKeyBoardData

{

int iValue;                 // see refer to SDK_NetKeyBoardValue

int iState;                 // see refer to SDK_NetKeyBoardState

};

### Network Alarm Information

/// netalarm

struct SDK_NetAlarmInfo

{

int iEvent;  //unused

int iState;  //status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

};

///< Alarm Center

struct SDK_NetAlarmCenterConfig

{

bool bEnable;		///< enable

char sAlarmServerKey[NET_NAME_PASSWORD_LEN];	///< alarm center type

///< server info

SDK_RemoteServerConfig Server;

bool bAlarm; //report alarm msg

bool bLog;   //report log msg

};

struct SDK_NetAlarmServerConfigAll

{

SDK_NetAlarmCenterConfig vAlarmServerConfigAll[NET_MAX_ALARMSERVER_TYPE];

};

// alarm center msg type

enum SDK_AlarmCenterMsgType

{

SDK_ALARMCENTER_ALARM,

SDK_ALARMCENTER_LOG,

};

// alarm status

enum SDK_AlarmCenterStatus

{

SDK_AC_START,

SDK_AC_STOP,

};

// alarm msg info

struct SDK_NetAlarmCenterMsg

{

CONFIG_IPAddress HostIP;		///< device ip

int nChannel;                   ///< channel

int nType;                      ///< see refer to AlarmCenterMsgType

int nStatus;                    ///< see refer to AlarmCenterStatus

SDK_SYSTEM_TIME Time;           ///< start time

char sEvent[NET_MAX_INFO_LEN];  ///< event

char sSerialID[NET_MAX_MAC_LEN]; ///< serial ID

char sDescrip[NET_MAX_INFO_LEN];  ///< description of event

};

### Storage Device Control Information

/// Storage Device Control

struct SDK_StorageDeviceControl

{

int iAction;	///< see enum SDK_StorageDeviceControlTypes

int iSerialNo;	///< disk No.

int iPartNo;    ///< Partition No.

int iType;		///< see enum SDK_StorageDeviceClearTypes or SDK_FileSystemDriverTypes

int iPartSize[2/*MAX_DRIVER_PER_DISK*/];	///< partition size

};

### RTSP

//RTSP

struct SDK_NetRTSPConfig

{

bool bServer;

bool bClient;

SDK_RemoteServerConfig Server;		///< server mode

SDK_RemoteServerConfig Client;		///< client mode

};

### MECG platform

//MECG platform

struct SDK_CONFIG_NET_MEGA

{

bool bEnable;

bool bNetManEnable;

CONFIG_IPAddress ServerIP;

int iServerPort;

char sDeviceId[32];

char sUserName[24];

char sPasswd[32];

int iMaxCon;

int iVideoPort;

int iAudioPort;

int iMsgPort;

int iUpdatePort;

};

### XINWANG platform

struct SDK_CONFIG_NET_XINGWANG

{

bool bEnable;

bool bSyncTime;

bool bSubStream;

CONFIG_IPAddress ServerIP;

int iServerPort;

int iDownLoadPort;

char sPasswd[32];

char szSID[32];

};

### SHISOU platform

struct SDK_CONFIG_NET_SHISOU

{

bool bEnable;

SDK_RemoteServerConfig Server;

char szSID[NET_MAX_USERNAME_LENGTH];

};

### VVEYE platform

struct SDK_CONFIG_NET_VVEYE

{

bool bEnable;

bool bCorpEnable;            //Only use Business Server

SDK_RemoteServerConfig Server;

char szDeviceName[NET_MAX_USERNAME_LENGTH];

};

### Media Packet Information

enum SERIAL_TYPE

{

RS232 = 0,

RS485 = 1,

};

enum MEDIA_PACK_TYPE

{

FILE_HEAD =	0,

VIDEO_I_FRAME = 1,

VIDEO_B_FRAME =	2,

VIDEO_P_FRAME = 3,

VIDEO_BP_FRAME = 4,

VIDEO_BBP_FRAME	= 5,

VIDEO_J_FRAME = 6,

AUDIO_PACKET = 10,

};

typedef struct

{

int		nPacketType;				// refer to MEDIA_PACK_TYPE

char*	pPacketBuffer;				// packet buffer

unsigned int	dwPacketSize;		// packet size

// absoluteness timestamp

int		nYear;						// year

int		nMonth;						// month

int		nDay;						// day

int		nHour;						// hour

int		nMinute;					// min

int		nSecond;					// sec

unsigned int 	dwTimeStamp;	    // opposite timestamp low bit（0~31）, unit:ms

unsigned int	dwTimeStampHigh;    // opposite timestamp low bit（32~63）, unit:ms

unsigned int   dwFrameNum;             //frame index

unsigned int   dwFrameRate;            //frames per sec

unsigned short uWidth;              // video width

unsigned short uHeight;             // video height

unsigned int       Reserved[6];

} PACKET_INFO_EX;

struct SDK_OEMInfo

{

int nOEMID;                     //OEM ID

char sCompanyName[NET_MAX_USERNAME_LENGTH]; //company name

char sTel[NET_MAX_USERNAME_LENGTH];         //telephone

char sAddr[NET_MAX_USERNAME_LENGTH];        //address

};

typedef struct __TransComChannel//TransComm

{

SERIAL_TYPE TransComType;//SERIAL_TYPE

unsigned int baudrate;

unsigned int databits;

unsigned int stopbits;

unsigned int parity;

} TransComChannel;

typedef enum SDK_State_Type

{

DEV_STATE_DDNS=0,

};

//Camera Parameter

//Exposure Config

struct SDK_ExposureCfg

{

int  level;    //the level

unsigned int leastTime;//the least time of auto Exposure

unsigned int mostTime; //the most time of auto Exposure

};

//gain config

struct SDK_GainCfg

{

int gain;    //the value of gain

int autoGain;//auto gain，0:close  1:open

};

//Network Camera Parameter

struct SDK_CameraParam

{

unsigned int whiteBalance;         //White Balance

unsigned int dayNightColor;        //Day and Night mode

int elecLevel;             //

unsigned int apertureMode;          //

unsigned int BLCMode;               //

SDK_ExposureCfg exposureConfig;     //exposure config

SDK_GainCfg     gainConfig;    //gain config

};

//All Camera Config

struct SDK_AllCameraParam

{

SDK_CameraParam vCameraParamAll[NET_MAX_CHANNUM];   //All Channel

};

//the Ability of exposure

struct SDK_CameraAbility

{

int  count;      //the speed number

unsigned int speeds[CAMERAPARA_MAXNUM]; //speed

int  reserve[15];//remain

};

### LocalPlay control

//本地播放控制

enum SDK_LoalPlayAction

{

SDK_Local_PLAY_PAUSE,		/*<! 暂停播放*/

SDK_Local_PLAY_CONTINUE,		/*<! 继续正常播放*/

SDK_Local_PLAY_FAST,	        /*<! 加速播放*/

SDK_Local_PLAY_SLOW,	        /*<! 减速播放*/

};

### Active Register Server

//Ther Callback data

struct H264_DVR_ACTIVEREG_INFO

{

char deviceSarialID[64];//Serials number

H264_DVR_DEVICEINFO deviceInfo;//device  Info

};

### SubConnectStyle

enum SubConnType

{

conn_realTimePlay=1,

conn_talk,

conn_playback

};

### ConnectStyle

enum SocketStyle

{

TCPSOCKET=0,

UDPSOCKET,

SOCKETNR

};

# API Definition

## SDK Initialization

- H264_DVR_API long H264_DVR_GetLastError();;

- API description: It is to return function failure code. when you failed to call the following interface, you can call this fucntion to get error code.

- Parameter: none

- Return: Please see refer to error code

- Reference API：

typedef void (__stdcall *fDisConnect)(long lLoginID, char *pchDVRIP, long nDVRPort, unsigned long dwUser);

- H264_DVR_API long H264_DVR_Init(fDisConnect cbDisConnect, unsigned long dwUser);

- API description：Initialize SDK, calling before all SDK function

- Parameter：

- cbDisConnect

- Disconnect callback function. It is to callback disconnect device excluding device logout successfully(call H264_DVR_Logout(), set it as 0 when forbid callbacking.

- [in]dwUser

- User data

- CallBack function Parameters：

- lLoginID

- Login handle

- pchDVRIP

- Device IP

- nDVRPort

- Port

- dwUser

- User data, just the same with the above user data you have input.

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_Cleanup

- CLIENT_API void H264_DVR_Cleanup ();

- API description：Clean up SDK and release occupied resource, calling after all SDK function.

- Parameter：none

- Return：none

- Reference API：H264_DVR_Init

## Get alarm status

typedef bool (__stdcall *fMessCallBack)(long lLoginID, char *pBuf,

unsigned long dwBufLen, long dwUser);

- H264_DVR_API bool H264_DVR_SetDVRMessCallBack(fMessCallBack cbAlarmcallback, unsigned long lUser);

- API description：Set device message callback fuction to get device current state. Callback order does not matter here. SDK default setting is not to callback.You need to call alarm message subscription interface H264_DVR_SetupAlarmChan().It applies to current defined alarm status.Device state is callbacked every second.每态次ENT_Cleanup0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000

- Parameter：

- cbAlarmcallback

- Message callback function.It is to callback device status (such as alarm status). When it is 0, system disables callback.

- [in] lUser

- user self-defined data

    - Callback function parameters:

- lLoginID

- Return value of H264_DVR_Login

- pBuf

Refer to see SDK_AlarmInfo

- dwBufLen

- pBuf length. Unit is byte.

- dwUser

- User self-defined data

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_SetupAlarmChan、H264_DVR_CloseAlarmChan

- H264_DVR_API long H264_DVR_SetupAlarmChan(long lLoginID);

- API description：Start listening device message. This function is to set callbacking device message or not. Message is callbacked from H264_DVR_SetDVRMessCallBack.

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_SetDVRMessCallBack，H264_DVR_CloseAlarmChan

- H264_DVR_API bool H264_DVR_CloseAlarmChan(long lLoginID);

- API description：Stop lisening one device

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_SetupAlarmChan

## Device Registration

- H264_DVR_API long H264_DVR_Login (char *sDVRIP, unsigned short wDVRPort, char *sUserName, char *sPassword, LPH264_DVR_DEVICEINFO lpDeviceInfo, int *error，SocketStyle socketTyle=TCPSOCKET);

- API description：Login. When device set the user as reuse(device default user such as admin can be reused.). this account can registed several time.

- Parameter：

- [in] sDVRIP

- device IP

- [in] wDVRPort

- device port

- [in] sUserName

- user name

- [in] sPassword

- password

- [out] lpDeviceInfo

- device property. it is a output parameter.

- [out] error

- (when the function returned successfully, the parameter is null.Please refer to error code.

- [in] socketTyle

- 登入类型 参见：SocketStyle

- Return：Return o if failed. Return device ID if succeeded. Using this value(device handle)all operations after successfuly log in can corresponding to the device.

- Reference API：H264_DVR_Logout

- H264_DVR_API long H264_DVR_LoginEx(char *sDVRIP, unsigned short wDVRPort, char *sUserName, char *sPassword, LPH264_DVR_DEVICEINFO lpDeviceInfo, int nType, int *error);

- API description：Register a user to device extension port. support one user specify device.

- Parameter：

- [in] sDVRIP

- device IP

- [in] wDVRPort

- device port

- [in] sUserName

- user name

- [in] sPassword

- password

- [out] lpDeviceInfo

- device property. it is a output parameter.

- [in] nType

- the type as follows:

enum LoginType

{

LOGIN_TYPE_GUI,			///< Local GUI

LOGIN_TYPE_CONSOLE,		///< Console

LOGIN_TYPE_WEB,			///< WEB

LOGIN_TYPE_SNS,			///< SNS

LOGIN_TYPE_MOBIL,		///< Mobile terminal

LOGIN_TYPE_NETKEYBOARD,	///< Netkeyboard

LOGIN_TYPE_SERVER,		///< Center servers

LOGIN_TYPE_AUTOSEARCH,	///< IP search tool

LOGIN_TYPE_UPGRADE,		///< Upgrade tool

LOGIN_TYPE_MEGAEYE,		///< Megaeye

LOGIN_TYPE_NR,

- };

- [out] error

- (when the function returned successfully, the parameter is null.Please refer to error code.

- Return: Return 0 if failed. Return device ID if succeeded.Using this value(device handle)all operations after successfully logged in can corresponding to the device.

- Reference API: H264_DVR_Logout

- H264_DVR_API long H264_DVR_Logout(long lLoginID)

- API description：Logout user

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_Login

## Real-time Monitor

- H264_DVR_API long H264_DVR_RealPlay(long lLoginID, LPH264_DVR_CLIENTINFO lpClientInfo);

- API description：Start real-time monitor.

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] lpClientInfo

Client information

- Return：Return real-time monitor handle if succeeded, return less than 0 if failed.

- Reference API：H264_DVR_StopRealPlay，H264_DVR_SetRealDataCallBack

- H264_DVR_API bool H264_DVR_StopRealPlay(long lRealHandle,void *hWnd==NULL);

- API description：Stop real-time monitor

- Parameter：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- [in]hWnd

- The handle of window that is playing

- Default NULL close all Windows

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_RealPlay

- H264_DVR_API long H264_DVR_PauseRealPlay(long lRealHandle, bool bPause);

- API description：Pause real-time monitor

- Parameter：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- [in] bPause

- Enable of Pause.0:Continue Play; 1:Pause

- Return：Succeeded: TRUE, Fail: FALSE

- H264_DVR_API bool H264_DVR_SetRealDataCallBack(long lRealHandle,fRealDataCallBack cbRealData, long dwUser);

typedef int(__stdcall *fRealDataCallBack) (long lRealHandle, long dwDataType, unsigned char *pBuffer,long lbufsize,long dwUser);

- API description：Set real-time monitor data callback and provides you with data from the device .When cbRealData is NULL, callback ends.

- Parameter：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- cbRealData

- It is a callback function to output the current real-time data from the device.

- [in]dwUser

- User data

            - Callback function parameters:

- lRealHandle

- Return value of H264_DVR_RealPlay

- dwDataType

- 0: original data

- 1: Frame data

- 2: yuv data

- 3: pcm audio data

- pBuffer

- call-back data. Everytime calling back data of different lengths according to the different data types (except type 0). Other data types are based on frames, every time it calls back one frame.

- dwBufSize

- length of callback data.(Unit:byte).

- dwUser

- User self-defined

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_RealPlay、H264_DVR_StopRealPlay

typedef int(CALL_METHOD *fRealDataCallBack_V2) (long lRealHandle, const PACKET_INFO_EX *pFrame, unsigned int dwUser);

- H264_DVR_API bool CALL_METHOD H264_DVR_SetRealDataCallBack_V2(long lRealHandle,fRealDataCallBack_V2 cbRealData, long dwUser);

- API description：Set real-time monitor data callback and provides you with data from the device .When cbRealData is NULL, callback ends.

- Parameters Description：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- cbRealData

- It is a callback function to output the current real-time data from the device.

- [in]dwUser

- User self-defined

- callback function description:

- lRealHandle

- Return value of H264_DVR_RealPlay

- pFrame

- Reference to PACKET_INFO_EX

- dwUser

- User self-defined

- Return：Succeeded: TRUE, Fail: FALSE

- H264_DVR_API bool H264_DVR_DelRealDataCallBack(long lRealHandle,fRealDataCallBack cbRealData, long dwUser);

- API  Description：Clear up call-function before call H264_DVR_StopRealPlay

- Parameter description：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- cbRealData

- It is a callback function to output the current real-time data from the device.

- [in]dwUser

- User self-defined

- Return：Succeeded: TRUE, Fail: FALSE

- H264_DVR_API bool CALL_METHOD H264_DVR_DelRealDataCallBack_V2(long lRealHandle,fRealDataCallBack_V2 cbRealData, long dwUser);

- API description：Clear up call-function before call H264_DVR_StopRealPlay Parameters description：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- cbRealData

- It is a callback function to output the current real-time data from the device.

- [in]dwUser

- User self-defined

- Return：Succeeded: TRUE, Fail: FALSE

## Playback And Download

- H264_DVR_API long H264_DVR_FindFile(long lLoginID, H264_DVR_FINDINFO* lpFindInfo, H264_DVR_FILE_DATA *lpFileData, int lMaxCount, int *findcount, int waittime = 2000);

- API description：Search record files

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in]lpFindInfo

- Search condition H264_DVR_FINDINFO

- [out]lpFileData

- Returned record file information.It is a H264_DVR_FILE_DATA

- structure array.

- [in]lMaxCount

- The length of lpFileData（Unit: BYTE，The value shall between 100~200*sizeof(H264_DVR_FILE_DATA)）

- [out]filecount

- Returned file amount; It is an output max parameter. You can only search the video files before buffer is full

- [in]waittime

- Waitting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：

- H264_DVR_Login,H264_DVR_PlayBackByName,H264_DVR_StopPlayBack,

- H264_DVR_PlayBackControl, H264_DVR_GetFileByName

- H264_DVR_API long H264_DVR_FindFileByTime(long lLoginID, SDK_SearchByTime* lpFindInfo, SDK_SearchByTimeResult *lpFileData, int waittime = 10000);

- API description：Search record files by time

- Parameters description：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] lpFindInfo

- Reference to SDK_SearchByTime

- [out] lpFileData

- Returned record file information.It is a H264_DVR_FILE_DATA

- structure array.

- [in]waittime

- Waitting time

- Return：Succeeded: TRUE, Fail: FALSE

typedef void(__stdcall *fDownLoadPosCallBack) (long lPlayHandle, long lTotalSize, long lDownLoadSize, long dwUser)

- H264_DVR_API long H264_DVR_PlayBackByName(long lLoginID, H264_DVR_FILE_DATA *sPlayBackFile, fDownLoadPosCallBack cbDownLoadPos, fRealDataCallBack fDownLoadDataCallBack, long dwDataUser);

- API description：Network playback. Please note, when you login one device, one channel can only play one record at one time, while multi-records of the same channel can’t be opened simutaniously.

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] sPlayBackFile

- Recorded file information return by H264_DVR_FindFile

- [in] cbDownLoadPos

- Now: It’s only for Get the end information,(Use

- H264_DVR_GetPlayPos to get the playing positon)

- [in] fDownLoadDataCallBack

- Video data call-back function

- [in]dwUserData

- User self-defined data

            - CallBack function：

- lPlayHandle

- Return value of H264_DVR_PlayBackByName

- dwTotalSize

- Current total play size, unit is KB.

- dwDownLoadSize

- Played size，unit is KB. When value is -1, it means cureent playback is over.

- dwUser

- User data, just the same with user data in the above.

- Return：Return network playbackID if succeeded, return 0 if failed.

- Reference API：

- H264_DVR_Login,H264_DVR_FindFile,H264_DVR_StopPlayBack,

- H264_DVR_PlayBackControl

typedef int(CALL_METHOD *fRealDataCallBack_V2) (long lRealHandle, const PACKET_INFO_EX *pFrame, unsigned int dwUser);

- H264_DVR_API long H264_DVR_PlayBackByName_V2(long lLoginID, H264_DVR_FILE_DATA *sPlayBackFile, fDownLoadPosCallBack cbDownLoadPos, fRealDataCallBack_V2 fDownLoadDataCallBack, long dwDataUser);

- API description：Network play back by name

- Parameters description：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] sPlayBackFile

- Recorded file information return by H264_DVR_FindFile

- [in] cbDownLoadPos

- Now: It’s only for Get the end information,(Use

- H264_DVR_GetPlayPos to get the playing positon)

- [in] fRealDataCallBack_V2

- Video data call-back function

- [in]dwUserData

- User data, just the same with user data in the above

- Call-function description.

- lRealHandle:

- Return value of H264_DVR_RealPlay

- pFrame:

- Reference to the structure of PACKET_INFO_EX

- dwUser:

- User data, just the same with user data in the above

- Call-function description.

- Return：Return network playbackID if succeeded, return 0 if failed.

- H264_DVR_API long H264_DVR_PlayBackByTime(long lLoginID, H264_DVR_FINDINFO* lpFindInfo, fDownLoadPosCallBack cbDownLoadPos, fRealDataCallBack fDownLoadDataCallBack, long dwDataUser);

- API description：(It’s old ,Please use the down function

- “H264_DVR_PlayBackByTimeEx”)

- Parameters description：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] lpFindInfo

- Reference to H264_DVR_FINDINFO

- [in] cbDownLoadPos

- Now: It’s only for Get the end information,(Use

- H264_DVR_GetPlayPos to get the playing positon)

- [in] fDownLoadDataCallBack

- Video data call-back function

- [in]dwUserData

- User data, just the same with user data in the above

- Call-function description.

- Return：Return network playbackID if succeeded, return 0 if failed

- H264_DVR_API long H264_DVR_PlayBackByTimeEx(long lLoginID, H264_DVR_FINDINFO*lpFindInfo,fRealDataCallBack cbDownLoadDataCallBack, long dwDataUser,fDownLoadPosCallBack cbDownLoadPos, long dwPosUser);

- API description：Record playback by time

- Parameters description：

- [in]lLoginID

- Return value of  H264_DVR_Login

- [in] lpFindInfo

- Reference to H264_DVR_FINDINFO

- [in] fRealDataCallBack

- Real-play call-function

- [in] dwDataUser

User data

- [in] cbDownLoadDataCallBack

- Now: It’s only for Get the end information,(Use

- H264_DVR_GetPlayPos to get the playing positon)

- [in] dwPosUser

- User data, just the same with user data in the above

- Call-function description.

- Return：Return network playbackID if succeeded, return 0 if failed

- H264_DVR_API bool H264_DVR_StopPlayBack(long lPlayHandle);

- API description：Stop playback

- Parameter：

- [in]lPlayHandle

- Playback handle, Such as the return value of H264_DVR_PlayBackByName

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_PlayBackByName

- H264_DVR_API long H264_DVR_GetFileByName(long lLoginID,H264_DVR_FILE_DATA *sPlayBackFile,char *sSavedFileName, fDownLoadPosCallBack cbDownLoadPos = NULL, long dwDataUser = NULL );

- API description：Download recorded files via the searched information

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] sPlayBackFile

- Recorded file information pointer.

- [in]sSavedFileName

- The name of file to save(full path).

- [in]cbDownLoadPos

- Download process calls user self-defined data.For download process callback fucntion, please refer to H264_DVR_GetDownloadPos

- [in]dwUserData

- Download process calls user self-defined data.

            - CallBack function：

- lPlayHandle

- Return value of H264_DVR_PlayBackByName

- dwTotalSize

- Current total play size, unit is KB.

- dwDownLoadSize

- Played size，unit is KB. When value is -1, it means cureent playback is over.

- dwUser

- User data, just the same with user data in the above.

- Return：Return download ID if succeeded. Return 0 if failed.

- Reference API：H264_DVR_StopGetFile、H264_DVR_GetDownloadPos

- H264_DVR_API long H264_DVR_GetFileByTime(long lLoginID, H264_DVR_FINDINFO* lpFindInfo, char *sSavedFileDIR, fDownLoadPosCallBack cbDownLoadPos = NULL ，bool bMerge = false)

- API description：Download recorded files via the searched information by time

- Parameters：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] lpFindInfo

- Reference to H264_DVR_FINDINFO

- [in] sSavedFileDIR

- Download the record to the directory

- cbDownLoadPos

- Download process calls user self-defined data.For download process callback fucntion, please refer to H264_DVR_GetDownloadPos

- [in] bMerge

- whether merging the download files to one file or not

- Return：Return download ID if succeeded. Return 0 if failed.

- H264_DVR_API bool H264_DVR_StopGetFile(long lFileHandle);

- API description：Stop downloading files.

- Parameter：

- [in]lFileHandle

- The return value of H264_DVR_GetFileByName

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_GetFileByName、H264_DVR_GetDownloadPos

- H264_DVR_API int H264_DVR_GetDownloadPos(long lFileHandle);

- API description：It is to get current downloading place for interfaces that do not need to show real-time download progress. It is similar to the download callback function.

- Parameter：

- [in]lFileHandle

- The return value of H264_DVR_GetFileByName

- Return：position(value between 0 to 100)

- Reference API：H264_DVR_GetFileByName、H264_DVR_StopGetFile

## Playback Control

- H264_DVR_API bool H264_DVR_PlayBackControl(long lPlayHandle, long lControlCode,long lCtrlValue);

- API description：Pause , Resume, Seek network playback

- Parameter：

- [in]lPlayHandle

- Play handle, return by H264_DVR_GetFileByName

- [in] lControlCode

enum SEDK_PlayBackAction

{

SDK_PLAY_BACK_PAUSE,		// Pause

SDK_PLAY_BACK_CONTINUE,		// Resume

SDK_PLAY_BACK_SEEK,			// Seek

SDK_PLAY_BACK_FAST,	        /*<! Fast */

SDK_PLAY_BACK_SLOW,	        /*<! Slow */

SDK_PLAY_BACK_SEEK_PERCENT, /*<! Position */

- };

- [in] lCtrlValue

- SDK_PLAY_BACK_SEEK （the pos of time(s)）

- SDK_PLAY_BACK_FAST (the fast value(one->four))

- SDK_PLAY_BACK_SLOW (the slow value(one->four))

- SDK_PLAY_BACK_SEEK_PERCENT (the pos of persent(1-100))

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_PlayBackByName、H264_DVR_StopPlayBack

## PTZ Control

- H264_DVR_API bool H264_DVR_PTZControl(long lLoginID,int nChannelNo, long lPTZCommand, bool bStop = false, long lSpeed = 4)

- API description：PTZ control

- Parameter:

[in]lLoginID

- Return value of H264_DVR_Login

[in] nChannelNo

Channel NO. start with 0

[out] lPTZCommand

Commands, see refer to PTZ_ControlType

[in] bStop

Whether stop or not. It applies to PTZ direction andlens operation. When you operate other functions, input this parameter as FALSE.

[out] lSpeed

Step/Speed. The value ranges from 1 to 8. 8 has the highest control capability(4 by default). dwStep is the preset value when you use preset function.

- Return: Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_Login，H264_DVR_RealPlay

- H264_DVR_API bool H264_DVR_PTZControlEx(long lLoginID,int nChannelNo, long lPTZCommand, long lParam1, long lParam2, long lParam3, bool bStop = false)

- API description：Expand toPTZ Setting，include Preset，cruise，Quick Location

- Parameters description:

[in]lLoginID

Return value of H264_DVR_Login

[in] nChannelNo

Device Channel No

[in] lPTZCommand

Control Type。Reference to PTZ_ControlType

[in] lParam1, lParam2, lParam3

Expand Ptz parameters

[in] bStop

Stop or not

- Return: Succeeded: TRUE, Fail: FALSE

- Tranditional Application：Expanded Ptz control，use before the Current Channel is open

## System Configuration

- H264_DVR_API long  H264_DVR_GetDevConfig(long lLoginID, unsigned long dwCommand, int nChannelNO, char * lpOutBuffer, unsigned long dwOutBufferSize, unsigned long* lpBytesReturned,int waittime = 1000);

- API description: Get device configuration information.

- Parameter:

- [in]lLoginID

- The return value of H264_DVR_Login

[in]dwCommand

Configuration type, Please refer to SDK_CONFIG_TYPE

[in]nChannelNO

Channel number. Set as -1 to configure all channels. The parameter is null if command does not need the channel number.

[out] lpOutBuffer

Receive data buffer pointer, buffer length depend on configuration structure size.

[in]dwOutBufferSize

Receive data buffer lenght(Unit:byte)

[out]lpBytesReturned

The data lenght actually received.

[in]waittime

Waiting time

- Return：Succeeded: >0, Fail: <0(Throught H264_DVR_GetLastError get more information)

- Reference API：H264_DVR_SetDevConfig

- H264_DVR_API long  H264_DVR_SetDevConfig(long lLoginID, unsigned long dwCommand, int nChannelNO, char * lpInBuffer, unsigned long dwInBufferSize, int waittime = 1000);

- API description: Set device configuration information

- Parameter:

- [in]lLoginID

- The return value of H264_DVR_Login

[in]dwCommand

Configuration type, Please refer to SDK_CONFIG_TYPE

[in]nChannelNO

Channel number. Set as -1 to configure all channels. The parameter is null if command does not need the channel number.

[in]lpInBuffer

Data buffer pointer

[in]dwInBufferSize

Data buffer lenght(unit is byte).

[in]waittime

Waitting time

- Return：Succeeded: >0, Fail: <0 (Throught H264_DVR_GetLastError get more information)

- Reference API：H264_DVR_GetDevConfig

## Log Management

- H264_DVR_API bool H264_DVR_FindDVRLog(long lLoginID, SDK_LogSearchCondition *pFindParam, SDK_LogList *pRetBuffer, long lBufSize, int waittime = 2000);

- API description：Log query

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] pFindParam

- Search condition, Please refer to SDK_LogSearchCondition

- [in] pRetBuffer

- The return of log information, Please refer to SDK_LogList

- [in] lBufSize

- The return length of log information

- [in]	waittime

- Waiting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：None

## Remote Control

- H264_DVR_API bool H264_DVR_ControlDVR(long lLoginID, int type, int waittime = 2000)

- API description：Reboot device and clear log

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] type

- 0: reboot device，1: clear log

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：None

## 3.11 Upgrade

typedef void(__stdcall *fUpgradeCallBack) (long lLoginID, long      lUpgradechannel, int nTotalSize, int nSendSize, long dwUser);

- H264_DVR_API long H264_DVR_Upgrade(long lLoginID, char *sFileName, int nType = 0, fUpgradeCallBack cbUpgrade = NULL, long dwUser = 0);

- API description：Upgrade device

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] sFileName

- The name of upgrade file

- [in] nType

- The type of upgrade file

enum UpgradeTypes

{

UPGRADE_TYPES_SYSTEM,	///< System

UPGRADE_TYPES_NR,

- };

            - CallBack function：

- fUpgradeCallBack

- Callback of upgrade progress, lUpgradechannel is the upgrade handle, return by H264_DVR_Upgrade

- nTotalSize

- Upgrade file lenght,(Unit: BYTE)

- nSendSize

- data length have been upgraded,(unit: BYTE)

- [in]dwUser

- User self-define data

- Return：Succeeded: return upgrade handle, Fail: FALSE

- Reference API:H264_DVR_GetUpgradeState,H264_DVR_CloseUpgradeHandle

- H264_DVR_API long H264_DVR_CloseUpgradeHandle(long lUpgradeHandle);

- API description：stop upgrade

- Parameter：

- [in]lUpgradeID

- The return value of H264_DVR_Upgrade

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API: H264_DVR_Upgrade

- H264_DVR_API int H264_DVR_GetUpgradeState(long lUpgradeHandle)

  - API description：get upgrade status

  - Parameter：

- [in] lUpgradeHandle

- The return value of H264_DVR_Upgrade

- Return：1: Succeeded 2: Upgrading, 3: FALSE

- Reference API: H264_DVR_Upgrade，H264_DVR_CloseUpgradeHand

- H264_DVR_API bool H264_DVR_SearchDevice(char* szBuf, int nBufLen, int* pRetLen,

int nSearchTime);

  - API description：Search device in LAN

  - Parameter：

[in] szBuf

Buffer to receive structure SDK_CONFIG_NET_COMMON,  return the a structure as long as it search a device.

[in] nBufLen

szBuf buffer length

[in] pRetLen

Return the structure total length of SDK_CONFIG_NET_COMMON

[in] nSearchTime

Waiting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

## Audio Talk

typedef void (__stdcall *pfAudioDataCallBack)(long lVoiceHandle, char *pDataBuf,

long dwBufSize, char byAudioFlag, long dwUser);

- H264_DVR_API long H264_DVR_StartVoiceCom_MR (long lLoginID, pfAudioDataCallBack pVcb,

Long  dwDataUser);

  - API description：Send audio talk request to device.

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] pVcb

Self-defined data callback interface.

[in] dwDataUser

Self-defined data. Returned to you via callback function

- Return：>0 audio talk handle, <= FALSE

- Reference API:

H264_DVR_VoiceComSendData, H264_DVR_StopVoiceCom, H264_DVR_SetTalkMode

- H264_DVR_API bool H264_DVR_VoiceComSendData (long lVoiceHandle, char *pSendBuf,

long lBufSize);

  - API description：Send user audio data to device.

  - Parameter：

[in] lVoiceHandle

Return value of H264_DVR_StartVoiceCom_MR

[in] pSendBuf

The audio data to be sent out.

[in] lBufSize

Audio data length to be sent out.(Unit:byte)

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

H264_DVR_StartVoiceCom_MR   H264_DVR_StopVoiceCom   H264_DVR_SetTalkMode

- H264_DVR_API bool H264_DVR_StopVoiceCom (long lVoiceHandle);

  - API description：Stop audio talk

  - Parameter：

[in] lVoiceHandle

The return value of H264_DVR_StartVoiceCom_MR

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

H264_DVR_StartVoiceCom_MR   H264_DVR_VoiceComSendData   H264_DVR_SetTalkMode

- H264_DVR_API bool H264_DVR_SetTalkMode (long lLoginID, SDK_AudioInFormatConfig* pTalkMode);

  - API description：set audio talk mode

  - Parameter：

    - [in] lLoginID

The return value of H264_DVR_Login

    - [in] pTalkMode

Mode of audio talk ,see refer to SDK_AudioInFormatConfig

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

H264_DVR_StartVoiceCom_MR   H264_DVR_VoiceComSendData   H264_DVR_StopVoiceCom

## Record Mode

- H264_DVR_API bool H264_DVR_StartDVRRecord(long lLoginID, int nChannelNo ,long lRecordType);

  - API description：set record mode

  - Parameter：

lLoginID

the return value of H264_DVR_Login

[in] nChannelNo

Channel No., -1 all of channel

[in] lRecordType

Record mode, see refer to SDK_RecordModeTypes

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  H264_DVR_StopDVRRecord

- H264_DVR_API bool H264_DVR_StopDVRRecord(long lLoginID, int nChannelNo);

  - API description：stop record

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] nChannelNo

Channel No., -1: all of channel

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  H264_DVR_StartDVRRecord

## Set System Time

- H264_DVR_API bool H264_DVR_SetSystemDateTime (long lLoginID, SDK_SYSTEM_TIME *pSysTime);

  - API description：set system time

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] pSysTime

System time, see refer to SDK_SYSTEM_TIME

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  None

## Get Device status

- H264_DVR_API bool H264_DVR_GetDVRWorkState(long lLoginID, SDK_DVR_WORKSTATE *pWorkState);

  - API description：Get device working status

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

    - [in] pWorkState

Structure of work staus, see refer to SDK_DVR_WORKSTATE

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  None

## Net Keyboard

- H264_DVR_API bool H264_DVR_ClickKey (long lLoginID, SDK_NetKeyBoardData *pKeyBoardData);

  - API description：send net keyboard message

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

    - [in] pKeyBoardData

Key value, see refer to SDK_NetKeyBoardData

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  None

## NetWork Alarm

- H264_DVR_API bool H264_DVR_SendNetAlarmMsg(long lLoginID, SDK_NetAlarmInfo *pAlarmInfo);

  - Description:Send network Alarminfo

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] pAlarmInfo

alarminfomation  see refer to：SDK_NetAlarmInfo

- Return：Succeeded: 1, Fail: 0

## AlarmCenter

typedef bool (__stdcall *fMessCallBack)(long lLoginID, char *pBuf,

unsigned long dwBufLen, long dwUser);

- H264_DVR_API bool H264_DVR_StartAlarmCenterListen(int nPort, fMessCallBack cbAlarmCenter, unsigned long dwDataUser);

- Description：open Alarm center

- Parameter：

[in] nPort

Netport

[in] cbAlarmCeter

Information(alarm)callback

[in] dwDataUser

userData

- Return：1: Successful   0 Failed

- H264_DVR_API bool H264_DVR_StopAlarmCenterListen();

- Description：close alarmcenter

- Return：1 Successful   0 Failed

## DiskManager

- H264_DVR_API int H264_DVR_StorageManage(long lLoginID, SDK_StorageDeviceControl *pStorageCtl);

  - Description：DiskManager

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] pStorageCtl

diskParam  see to：SDK_StorageDeviceControl

- Return：Larger then 0  Successful   oter: Failed；

## Capture

- H264_DVR_API bool H264_DVR_CatchPic(long lLoginID, int nChannel, char *sFileName);

  - Description：capture

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in]nChannel

Device channel

[in] sFileName

filename,must be all path

  - Return：1 Successful   0 falied

- H264_DVR_API  bool H264_DVR_CatchPicInBuffer(long lLoginID, int nChannel, char *pBuffer, int nBufLen, int *pPicLen);

  - Description：capture

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in]nChannel

Device channel

[out] pBuffer

The buffer of receive capture info

[in] nBufLen

The buffer length

[out] pPicLen

The real length

  - Return：1 Successful   0 falied

## Pass through channel 232,485

- H264_DVR_API bool H264_DVR_SerialWrite(long lLoginID, SERIAL_TYPE nType, char *pBuffer, int nBufLen);

  - Description：through of 232，485

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] nType

Set to SERIAL_TYPE

[int] pBuffer

Data buffer

[in] nBufLen

Data length

  - Return：1 Successful   0 falied

- H264_DVR_API bool H264_DVR_SerialRead(long lLoginID, SERIAL_TYPE nType, char *pBuffer, int nBufLen, int *pReadLen);

  - Description：through of 232，485

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] nType

See to SERIAL_TYPE

[out] pBuffer

The data buffer

[in] nBufLen

The buffer length

[out] pReadLen

The read length

  - Return：1 Successful   0 falied

## Get device information from DDNS

  - H264_DVR_API int H264_DVR_GetDDNSInfo(SearchMode &searchmode, DDNS_INFO *pDevicInfo, int maxDeviceNum, int &nretNum)

  - Description：Get device information from DDNS

  - Parameter：

[in] searchmode

Search information

[in] pDevicInfo

Device information buffer

[in] maxDeviceNum

The max devicd num

[out] nretNum

The finded device num

  - Return： >=0 Successful   <0 failed

## Enforcement I frame

- H264_DVR_API bool H264_DVR_MakeKeyFrame(long lLoginID, int nChannel, int nStream);

  - Description： Enforcement I frame

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in]nChannel

Device channel

[in] nStream

Stream type 0:main stream，1:sub stream

  - Return：1 successful   0 falied

## Setting connect timeout and retry  times

- H264_DVR_API bool CALL_METHOD H264_DVR_SetConnectTime(long nWaitTime, long nTryTimes);

  - Description：Setting connect timeout and retry  times

  - Parameter：

[in] nWaitTime

Connect timeout，default 5000ms,

[in] nTryTimes

- Connect times default three times

  - Return：1 successful   0 falied

## Open transparent channel

typedef void (CALL_METHOD *fTransComCallBack) (long lLoginID, long lTransComType, char *pBuffer, unsigned long dwBufSize, unsigned long dwUser);

- 1. H264_DVR_API bool H264_DVR_OpenTransComChannel (long lLoginID, TransComChannel *TransInfo, fTransComCallBack cbTransCom, unsigned long lUser);

  - CallBack function Parameters：

lLoginID

Login handle：H264_DVR_Login return

lTransComType

The type of serial port（485,232）reference the type  in “TransComChannel”

pBuffer

callBack data

dwBufSize

The length of buffer

dwUser

user’s data

  - Description：Open transparent window，Set callBack function

  - Parameter：

[in] lLoginID

- Login handle：H264_DVR_Login return

- [in] Transinfo

Configuration information of transparent window

[in] cbTransCom

callBack Function

[in] lUser

User’s data

  - Return：1 successful   0 falied

## Close transparent channel

- H264_DVR_API bool H264_DVR_CloseTransComChannel (long lLoginID, SERIAL_TYPE nType);

  - API Description: close the designated transparent channel

  - Parameter：

[in] lLoginID

- Return value of H264_DVR_Login

[in] nType

The type of transparent channel

  - Return：1 successful ;0 falied

## Get Device Status

- H264_DVR_API bool CALL_METHOD H264_DVR_GetDeviceState(long lLoginID, SDK_State_Type type, char *pState);

  - API description：Get Device Status

  - Parameters description：

[in] lLoginID

Return value of H264_DVR_Login

[in] nType

Reference to SDK_State_Type

[in] pState

Data Callback

  - Return: 1:successful;0:failed

## Get user operate interface

- H264_DVR_API bool CALL_METHOD H264_DVR_CatchPicUI(long lLoginID,char *saveFileName,int type=1)

  - API description：Get Device user operate interface

  - Parameters description：

[in] lLoginID

Return value of H264_DVR_Login

[in] saveFileName

The load of file that to saveing

[in] type

The type of picture(full or active picture ) 1:fullPicture 2:activePicture;

default is 1;

  - Return: 1:successful;0:failed

## Client-side Recording

- H264_DVR_API bool H264_DVR_StartLocalRecord（long lRealHandle,char*szSaveFileName,long type）

  - API description：On ther PC side to preview record

  - Parameters description：

[in] lRealHandle

H264_DVR_RealPlay Returned

[in] szSaveFileName

The FileName of Saved path

[in] type

Video type：（0：Filename suffix；2：Filename suffix.avi），default:0；

  - Return：1 successful   0 failed

- H264_DVR_API bool H264_DVR_StopLocalPlay(long lPlayHandle)

  - API description：Stop the PC video

  - Parameters description：

[in] lRealHandle

H264_DVR_RealPlay Returned

  - Return：1 successful   0 failed

## Open talking（2）

- H264_DVR_API long  H264_DVR_StartLocalVoiceCom(long lLoginID)

  - API description：open talking，H264_DVR_StopVoiceCom to stop talking

  - Parameters description：

[in] lLoginID

Returned value of H264_DVR_Login

  - Return：failed return 0，successful return talking ID，As  the parameter of 264_DVR_StopVoiceCom。

## Client-side Audio

- H264_DVR_API bool H264_DVR_OpenSound(long lHandle)

  - API description：Open the audio of video

  - Parameters description：

[in] lHandle

The returned value of H264_DVR_RealPlay or H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

  - Return：1 successful   0 failed

- H264_DVR_API bool H264_DVR_CloseSound(long lHandle)

  - API description：Close the audio of vido

  - Parameters description：

[in] lHandle

The returned value of H264_DVR_RealPlay or H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

  - Return：1 successful   0 failed

## Client-side cath picture

- H264_DVR_API bool H264_DVR_LocalCatchPic(long lHandle,char*szSaveFileName)

  - API description：PC side catch picture

  - Parameters description：

[in] lHandle

The returned value of H264_DVR_RealPlay or H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

[in] szSaveFileName

Saved path

  - Return：1 successful   0 failed

## Playing Position

- H264_DVR_API float H264_DVR_GetPlayPos(long lPlayHandle)

  - API description：Get the position of palyback or  Playing file

  - Parameters description：

[in] lPlayHandle

The return value of H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

  - Return： percentage of playing

- H264_DVR_API bool H264_DVR_SetPlayPos(long lPlayHandle,float fRelativPos)

  - API description：Set the position of palyback or Playing file

  - Parameters description：

[in] lPlayHandle

The returned value of H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

[in] fRelativPos

percentage of playing

  - Return：1 successful   0 failed

## Set information frame callback

- H264_DVR_API bool CALL_METHOD H264_DVR_SetInfoFrameCallBack(long lPlayHandle,InfoFramCallBack callback,long user)

prototype of callback function:

typedef void (*InfoFramCallBack)(long lPlayHand, long nType, LPCSTR pBuf,long nSize, long nUser);( nType:（0x03 :gprs information frame）)

  - API description：Set up information frame callback

  - Parameters description：

[in] lPlayHandle

The returned value of H264_DVR_RealPlay or H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

[in] callback

Call back function

[in] user

User data

  - Return：1 successful   0 failed

## The Color of Client-side

- H264_DVR_API bool CALL_METHOD H264_DVR_LocalGetColor(long lHandle, DWORD nRegionNum, LONG *pBrightness, LONG *pContrast, LONG *pSaturation, LONG *pHue)

  - API description：Get the information of color

  - Parameters description：

[in] lHandle

The returned value of H264_DVR_RealPlay or H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

[in] nRegionNum

region（current no used ）

[in] pBrightness

brightness

[in] pContrast

contrast

[in] pSaturation

saturation

[in] pHue

hue

  - Return：1 successful   0 failed

- H264_DVR_API bool CALL_METHOD H264_DVR_LocalSetColor(long lHandle, DWORD nRegionNum, LONG nBrightness, LONG nContrast, LONG nSaturation, LONG nHue)

  - API description：Set the information of color

  - Parameters description：

[in] lHandle

The returned value of H264_DVR_RealPlay or H264_DVR_StartLocalPlay or H264_DVR_PlayBackByName or H264_DVR_PlayBackByTimeEx

[in] nRegionNum

Region of color（current no used）

[in] pBrightness

brightness

[in] pContrast

contrast

[in] pSaturation

saturation

[in] pHue

hue

  - Return：1 successful   0 failed

## Play Client-side file

- H264_DVR_API long  H264_DVR_StartLocalPlay(char*pFileName,void* hWnd,fPlayDrawCallBack drawCallBack=NULL,long user=NULL)

prototype of callback function：

typedef void (CALL_METHOD * fPlayDrawCallBack)(long lPlayHand,HDC hDc,long nUser);

  - API description：play local file

  - Parameters description：

[in] pFileName

fileName

[in] hWnd

The handle of play window

[in] drawCallBack

Overlay rendering  callback function

[in] user

User data

  - Return：0 failed，successful:returned played ID(Local playing handle)

- H264_DVR_API bool H264_DVR_StopLocalPlay(long lPlayHandle)

  - API description：Stop PC side Playing

  - Parameters description：

[in] lPlayHandle

H264_DVR_StartLocalPlay Return

  - Return：1 successful   0 failed

- H264_DVR_API bool H264_DVR_SetFileEndCallBack(long lPlayHandle,fLocalPlayFileCallBack callBack,long user)

prototype of callback function：

typedef void (* fLocalPlayFileCallBack)(long lPlayHand, long nUser);

  - API description： Set callback function  of  ther end of LoalPlaying

  - Parameters description：

[in] lPlayHandle

H264_DVR_StartLocalPlay Return

[in] callBack

callBack function

[in] user

User data

  - Return：1 successful   0 failed

- H264_DVR_API bool CALL_METHOD H264_DVR_LocalPlayCtrl(long lPlayHandle,SDK_LoalPlayAction action,long lCtrlValue)

  - API description：Control localPlaying

  - Parameters description：

[in] lPlayHandle

H264_DVR_StartLocalPlay Return

[in] action

see：SDK_LoalPlayAction

[in] lCtrlValue

Fast playing（1,2,3,4），and slow playing（1,2,3,4）

## Binding local IP

- H264_DVR_API bool CALL_METHOD H264_DVR_SetLocalBindAddress(char*szIP)

  - API description：Set the binding of IP address (in the network card, can specify the binding of IP address)

  - Parameters description：

[in] szIP

The IP address of the binding

  - Return：1 successful   0 failed

## Upload data set callbacks

- H264_DVR_API bool CALL_METHOD H264_DVR_StartUploadData(long lLoginID,UploadDataType upLoadType,fUploadDataCallBack callBack,long lUser);

  - API description：Open Upload data, at present,only have vehicle-mounted type

  - Parameters description：

[in] lLoginID

Login handle

[in] upLoadType

Upload data type

[in] callBack

Upload data callback

[in] lUser

User data

  - Return：1 successful   0 failed

- H264_DVR_API bool CALL_METHOD H264_DVR_StopUploadData(longlLoginID,UploadDataType upLoadType);

  - API description：StopUploadData

  - Parameters description：

[in] lLoginID

Login handle

[in] upLoadType

Login type of data

Return：1 successful   0 failed

## Support advice to Active register

- 264_DVR_API bool CALL_METHOD H264_DVR_StartActiveRigister(int nPort, MessCallBack cbFunc, unsigned long dwDataUser);

  - API description：Beginning active registered monitor

  - Parameters description：

[in] nPort

Monitor port

[in] cbFunc

Device registered callback (callback parameters in: equipment handle is equivalent to the H264_DVR_Login login Return, Device information H264_DVR_ACTIVEREG_INFO buf)

[in] dwDataUser

User data

  - Return：1 successful   0 failed

- H264_DVR_API bool CALL_METHOD H264_DVR_StopActiveRigister()

  - API description：Stop to register the monitor

  - Return：1 successful   0 failed

## Set the sub connection broken callback

- 264_DVR_API long CALL_METHOD H264_DVR_SetSubDisconnectCallBack(fSubDisConnect callBack,DWORD userData);

  - API description：Set the Sub connection callback

  - Parameters description：

[in] callBack

The callBack Function （In Paramenters type is : SubConnType）

[in] userData

UserData

  - Return：1 successful   0 failed

## Set time interval of heartbeat and time interval of disconnect

- H264_DVR_API long CALL_METHOD H264_DVR_SetKeepLifeTime(long lLoginID, unsigned int perKeeplifeTime, unsigned int detectDisconTime)

  - API description：Set time interval of heartbeat And time interval of disconnect(from  the last time when you receive the heartbeat )

  - Parameters description：

[in] lLoginID

Login handle

[in] perKeeplifeTIme

Time interval of heartbeat

[in] detectDisconTime

Time interval of disconnect

  - Return：1 successful   0 failed